- Compiler

[Source]

This is used to display detailed information on the options for controlling preprocessor and make changes to the settings.

Include file directories: This option specifies the name of the path to the folder that stores the include file

Pre-include files: This option includes the specified file contents at the head of the compiling unit.

Defines: This option provides the same function as #define specified in the source file.

Invalidates the predefined macro: This option invalidates the predefined macro of <macro name>.

File inline path: This option specifies the name of the path where a file for inter-file inline expansion is stored.

The file for inter-file inline expansion is searched for in the order of the folders specified by this option and the current folder.

## (2) [Source]/[Source file]

This is used to display detailed information on the character code options and make changes to the settings.

C: This option specifies the language of the C source file.

C++: This option specifies the language of the C++ source file.

Input character code: This option specifies the character code to handle the characters in strings, character constants, and comments.

Interchangeability check: This option checks the specified options and source file parts which will affect the compatibility when this compiler uses a C/C++ source file that has been coded for the R8C and M16C family C compilers, H8, H8S, and H8SX family C/C++ compilers, and SuperH family C/C++ compilers

Allow comment nest: The nested comments are allowed to be written in the source file

## (3) [Object]

This is used to display detailed information on the output-related options and make changes to the settings.

Generate debug information: The debugging information necessary for C-source debugging is output. This option is valid even when an optimize option is specified.

Output file type: This option specifies the output file type.

Output assembly source file: This option outputs the assembly source file generated during compilation.

Output directory: The folder which serves as the origin of the folder where the object file created after compile will be saved is displayed. A folder having a relative path equivalent to that from the project folder to the folder holding the source file is created in the origin folder. After that, the object file is output to the folder.

Output character code: This option specifies the character code to output characters in strings and character constants.

Suppress DIVS, DIVU, and FDIV instruction generation: This option generates code in which no DIV, DIVU, or FDIV instructions are used for division and modular division operations in the program.This option calls the equivalent runtime functions instead of DIV, DIVU, or FDIV instructions. This may lower code efficiency in terms of required ROM capacity and speed of execution.

Section name of program area, constant area, initialized data area, uninitialized data area, switch statement branch table area, literal area: This option specifies the section name of a program area, constant area, initialized data area, uninitialized data area, switch statement branch table area, literal area

Allocates const qualified variable to 4 byte boundary alignment sections: This option allocates const qualified variables to 4-byte boundary alignment sections.

Allocates initialized variable to 4 byte boundary alignment sections: This option allocates initialized variables to 4-byte boundary alignment sections.

Allocates uninitialized variable to 4 byte boundary alignment sections: This option allocates uninitialized variables to 4-byte boundary alignment sections.

Allocates switch statement branch table to 4 byte boundary alignment sections: This option allocates switch statement branch tables to 4-byte boundary alignment sections.

Select adjustment for instruction in branch: This option aligns instructions at branch destinations.

## (4) [List]

This is used to display detailed information on the list options and make changes to the settings.

This category can be used when [Machine code (*.obj)] is selected for [Object] -> [Output file type].

Generate list file: This option outputs a source list file.

C/C++ source file: This option outputs the C/C++ source file to the source list file.

False line in conditional assembly: This option outputs also the statements for which the specified condition is not satisfied in conditional assembly to the source list file.

Before .DEFINE replacement: This option outputs the information before .DEFINE replacement to the source list file.

After macro expansion: This option outputs the assembler macro expansion statements to the source list file.

## (5) [Optimize]

This is used to display detailed information on the optimization options and make changes to the settings.

Generate file for inter-module optimization: This option generates the additional information for inter-module optimization in the output file.

At linkage, inter-module optimization is applied to files for which this option has been specified.

Optimize level: This option specifies the optimization level.

Optimize type: This option controls the optimization priority, either optimization for speed or optimization for size.

Perform global optimization: This option applies global optimization.

## (6) [Optimize]/[Advanced]

This is used to display detailed information on the other optimization options and make changes to the settings.

Optimization for access to external variables: This option optimizes accesses to global variables.

When the map option is specified, a base address is set by using an external symbol-allocation information file created by the optimizing linkage editor, and a code that uses addresses relative to the base address for accesses to global or static variables is generated.

Switch statement: This option specifies the expansion method of the switch statement.

Division by constant: Calculations for division and remainders of integer constants in the source file are converted into sequences of multiplication or bitwise operation (shift or bitwise AND operations) instructions.

Constant multiplication that can be performed through only shift operations and division and residue that can be performed through only bitwise AND operations cannot be controlled.

Library function: All library functions are called.

Schedule instructions: Instructions are scheduled taking into consideration pipeline processing.

Divide the optimization: The optimizing ranges of the large-size function are divided into many sections before compilation.

Use this option at performance tuning because it affects the object performance depending on the program.

Use floating point arithmetic instructions: A code using FPU instructions is generated.

Loop expansion: This option specifies whether to optimize loop expansion.

Expansion maximum number: The maximum expansion factor can be specified by <numeric value>.

Perform inline expansion automatically: The compiler automatically performs inline expansion.

Maximum increasing rate of function size: To specify the allowed increase in the function's size due to the use of inline expansion.

Performs the constant propagation of const qualified external variables: The constant propagation is performed even for const qualified global variables.

Treat global variables as volatile qualified: All external variables are handled as if they were volatile qualified.

Accordingly, the access count and access order for external variables are exactly the same as those written in the C/C++ source file.

Performs optimization considering the type of the data indicated by the pointer: This option selects whether to perform optimization with consideration for the type of the data indicated by the pointer.

## (7) [Miscellaneous]

This is used to display detailed information on the miscellaneous options and make changes to the settings.

Approximate a floating-point constant division: This option converts divisions of floating-point constants into multiplications of the corresponding reciprocals as constants.

Enable register declaration: This option allocates preferentially the variables with register storage class specification to registers.

Not check the range in conversion between floating- point number and integer: This option omits part of the type conversion processing for the floating type.

Change operation order for floating-point expression aggressively: This option optimizes modification of the operation order in a floating-point expression.

Detect stack overflow: This option generates codes for detecting stack overflow at the entry and exit of a function.

Value to be embedded for detecting stack overflow: This option specifies the value to be embedded to detect stack overflow.

Enable information-level message output: This option outputs the information-level messages.

The output of messages from the assembler or optimizing linkage editor cannot be controlled by this option.

Suppress number of information-level message: When an error number is specified, the output of the specified information-level message will be disabled. This option is only specifiable for the messages with error numbers 0510000 to 0549999 (including the component number).

Changes warning-level message to information-level message: This option changes warning-level messages to information-level messages.

Error number of warning-level message: Specify the error number for the warning-level message to be changed to an information-level message.

Changes information-level message to warning-level message: This option changes information-level messages to warning-level messages.

Error number of information-level message: Specify the error number for the information-level message to be changed to a warning-level message.

Changes information-level and warning-level message to error-level message: This option changes information-level and warning-level messages to error-level messages.

Error number of information-level and warning-level message: Specify the error number for the information-level message or warning-level message to be changed to an error-level message.

## (8) [User]

This is used to display detailed information on other items regarding the compiler and make changes to the settings.

User-defined options

## (9) [CPU]

This is used to display detailed information on the options for controlling CPU and make changes to the settings.

Instruction-set architecture: This option is used to select an instruction-set architecture (RXv1 or RXv2) for use in generating instruction codes.

CPU type: This option specifies the microcontroller type for the instruction code to be generated.

Data Endian: Select endian type for data.

Fast interrupt register: This option specifies the general registers which are to be used only in fast interrupt functions (functions that have the fast interrupt setting (fint) in their interrupt specification defined by #pragma interrupt).

ROM: This option specifies the general register used as a fixed base address throughout the program.

RAM: This option specifies the general register used as a fixed base address throughout the program.

Address Register: This option specifies the general register used as a fixed base address throughout the program.

| (Base register) Address (HE | This option Specify the address to be used in conjunction with the address register above. This corresponds to the -base option of the ccrx, asrx and lbgrx command. |  |
| --- | --- | --- |
|  | Default | 00000000 (hexadecimal number) |
|  | How to change | Specify the hexadecimal value from 0x0 to 0xFFFFFFFF. |
|  | Remark | This property can be used when the other than "None" is selected for [Address Register]. |

## (10) [CPU]/[Advanced]

This is used to display detailed information on the other options for controlling CPU and make changes to the settings.

| Patch code generation | This option is used to avoid a problem specific to the CPU type. This corresponds to the -patch option of the ccrx, asrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | None |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | None | --- | --- |
|  |  | RX610 | -patch=rx610 | The MVTIPL instruction which causes a problem in the RX610 Group is not used in the generated code. |
|  | Remark | None |  |  |

| Round to | This option specifies the rounding method for floating-point constant operations. This option does not affect the method of rounding for floating-point operations during program execution. This corresponds to the -round option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Nearest |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Zero | -round=zero | Values are rounded to zero. |
|  |  | Nearest | -round=nearest | Values are rounded to the nearest value. |
|  | Remark | None |  |  |

| Precision of double | This option controls whether or not the double type and long double type are changed to the float type. This corresponds to the -dbl_size option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Single precision |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Single precision | -dbl_size=4 | The option changes the type to the float type. |
|  |  | Double precision | -dbl_size=8 | The option does not change the type to the float type. |
|  | Remark | None |  |  |

| Sign of char | char type values are handled as signed. This corresponds to the -signed_char, -unsigned_char option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | unsigned |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | signed | -signed_char | char type values are handled as signed. |
|  |  | unsigned | -unsigned_char | char type values are handled as unsigned. |
|  | Remark | None |  |  |

| Sign of bit field | This option controls the type of bitfields. This corresponds to the -signed_bitfield, -unsiged_bitfield option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | unsigned |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | signed | -signed_bitfield | The value is handled as signed. |
|  |  | unsigned | -unsigned_bitfield | The value is handled as unsigned. |
|  | Remark | None |  |  |

| Allocate from lower bit | This option specifies the order of bit-field members. This corresponds to the -bit_order option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Lower bit |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Upper bit | -bit_order=left | The members are allocated from the upper bit. |
|  |  | Lower bit | -bit_order=right | The members are allocated from the lower bit. |
|  | Remark | None |  |  |

| Width of divergence of function | This option specifies the branch width. This corresponds to the -branch option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | 24 bit |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | 16 bit | -branch=16 | The program is compiled with a branch width within 16 bits. |
|  |  | 24 bit | -branch=24 | The program is compiled with a branch width within 24 bits. |
|  |  | 32 bit | -branch=32 | The branch width is not specified. |
|  | Remark | None |  |  |

| (Other CPU options) Denormalized number allowed as result | This option specifies the operation when denormalized numbers are used to describe floating-point constants. This option does not affect the handling of denormalized numbers in floating-point operations during program execution. This corresponds to the -denormalize option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -denormalize=on | The denormalized numbers are handled as they are. |
|  |  | Unchecked | -denormalize=off | The denormalized numbers are handled as zero. |
|  | Remark | None |  |  |

| (Other CPU options) Replace from int with short | Before compilation, the int type is replaced with the short type and the unsigned int type is replaced with the unsigned short type in the source file. I INT_MAX, INT_MIN, and UINT_MAX of limits.h are not converted by this option. This option is invalid during C++ and EC++ program compilation. This corresponds to the -int_to_short option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -int_to_short | Before compilation, the int type is replaced with the short type and the unsigned int type is replaced with the unsigned short type in the source file. |
|  |  | Unchecked | --- | Before compilation, the int type is not replaced with the short type and the unsigned int type is not replaced with the unsigned short type in the source file. |
|  | Remark | None |  |  |

| (Other CPU options) Enum size is made the smallest | This option processes the enumerated data qualified by enum as the minimum data type with which the enumeration value can fit in. This corresponds to the -auto_enum option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -auto_enum | This option processes the enumerated data qualified by enum as the minimum data type with which the enumeration value can fit in. |
|  |  | Unchecked | --- | This option is to process the enumeration type size as the signed long type. |
|  | Remark | None |  |  |

| (Other CPU options) Pack structures, unions and classes | This option specifies the boundary alignment value for structure members and class members. This corresponds to the –pack, -unpack option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -pack | Assume the boundary alignment value for structure members is 1 |
|  |  | Unchecked | -unpack | The boundary alignment value for structures and classes equals the maximum boundary alignment value for members. |
|  | Remark | None |  |  |

| (Other CPU options) Use try, throw and catch of C++ | The C++ exceptional handling function (try, catch, throw) is enabled. The code performance may be lowered. This corresponds to the -exception, -noexception option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -exception | The C++ exceptional handling function (try, catch, throw) is enabled. |
|  |  | Unchecked | -noexception | The C++ exceptional handling function (try, catch, throw) is disabled. |
|  | Remark | None |  |  |

| (Other CPU options) Use dynamic_cast and typeid of C++ | This option enables or disables runtime type information. This corresponds to the -rtti option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -rtti=on | The dynamic_cast and typeid are enabled. |
|  |  | Unchecked | -rtti=off | The dynamic_cast and typeid are disabled. |
|  | Remark | None |  |  |

| (Other CPU options) Save and restore ACC using the interrupt function | This option generates the saved and restored code of the accumulator (ACC, ACC0, ACC1) for interrupt functions. This corresponds to the -save_acc option of the ccrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -save_acc | This option generates the saved and restored code of the accumulator (ACC, ACC0, ACC1) for interrupt functions. |
|  |  | Unchecked | --- | It does not generate the saved and restored code of the accumulator (ACC, ACC0, ACC1) for interrupt functions. |
|  | Remark | None |  |  |

## (11) [PIC/PID]

This is used to display detailed information on the PIC/PID options and make changes to the settings.

| Enable the PIC function | This option generates code with the program section as PIC (position independent code). In PIC, all function calls are performed with BSR or BRA instructions. When acquiring the address of a function, a relative address from the PC should be used. This allows PIC to be located at a desired address after linkage. This option corresponds to the -pic option of the ccrx, asrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Disable |  |  |
|  | How to change | Select from the drop-down list. |  |  |
|  | Explanation in detail | Enable | -pic | This option generates code with the program section as PIC (position independent code). |
|  |  | Disable | --- | This option does not generate code with the program section as PIC (position independent code). |
|  | Remark | None |  |  |

| Enable the PID function | The constant area sections C, C_2, and C_1, the literal section L, and the switch statement branch table sections W, W_2, and W_1 are handled as PID (position independent data). PID can be accessed through a relative address from the PID register. This allows PID to be located at a desired address after linkage. A single general register is used to implement the PID function. This option corresponds to the -pid option of the ccrx, asrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes (The maximum bit width of the offset: 16 bits) | -pid=16 | The constant area sections C, C_2, and C_1, the literal section L, and the switch statement branch table sections W, W_2, and W_1 are handled as PID (position independent data). The maximum bit width of the offset when accessing the constant area section from the PID register as 16 bits. Enable the PID function. 16-bit (64 Kbytes to 256 Kbytes) addressing mode is supported. |
|  |  | Yes (The maximum bit width of the offset: No limitation) | -pid=32 | The constant area sections C, C_2, and C_1, the literal section L, and the switch statement branch table sections W, W_2, and W_1 are handled as PID (position independent data). The maximum bit width of the offset when accessing the constant area section from the PID register as 32 bits. |
|  |  | No | --- | The constant area sections C, C_2, and C_1, the literal section L, and the switch statement branch table sections W, W_2, and W_1 are not handled as PID (position independent data). |
|  | Remark | This property can be used when "None" is selected for [CPU] -> [(Base register) ROM] and the box for [Uses the PID register for code generation] is not checked. |  |  |

| Uses the PID register for code generation | The generated code uses the PID register. This option corresponds to the -nouse_pid_register option of the ccrx, asrx and lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Checked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | --- | The generated code uses the PID register. |
|  |  | Unchecked | -nouse_pid_register | The generated code does not use the PID register. |
|  | Remark | None |  |  |

## (12) [MISRA C Rule Check]

This is used to display detailed information on the MISRA C options and make changes to the settings.

| MISRA-C specification | This option specifies the MISRA-C specification to be conformed to. This corresponds to the -misra2004 and -misra2012 option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | MISRA-C:2012 |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | MISRA-C:2012 | -misra2012 | This option selects MISRA-C:2012. |
|  |  | MISRA-C:2004 | -misra2004 | This option selects MISRA-C:2004. |
|  | Remark | “MISRA-C:2012Ecan be specified when CC-RX V2.04.00 or a later version is used. |  |  |

| Apply rule | This option checks source code against the MISRA-C:2004 rules. A message is output if the item specified for the check. Specifying this option in the standard-version compiler will cause an error. This corresponds to the -misra2004 options of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Not apply rule |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Apply all rules | -misra2004=all | The compiler checks the source code against all of the rules that are supported. |
|  |  | Apply specified rules | -misra2004=apply | The compiler checks the source code against the rules with the selected numbers. |
|  |  | Ignore specified rules | -misra2004=ignore | The compiler checks the source code against the rules other than those with the selected numbers. |
|  |  | Apply all required rules | -misra2004=required | The compiler checks the source code against the rules of the "required" type. |
|  |  | Apply specified required rules | -misra2004=required_add | The compiler checks the source code against the rules of the "required" type and the rules with the selected numbers. |
|  |  | Ignore specified required rules | -misra2004=required_remove | The compiler checks the source code against the rules other than those with the selected numbers among the rules of the "required" type. |
|  |  | Apply all rules in the specified file | -misra2004=<file name> | The compiler checks the source code against the rules with the numbers written in the specified file. |
|  |  | Not apply rule | --- | The source code is not checked against the MISRA-C: 2004 rules. |
|  | Remark | This property can be used when CC-RX V1.02.00 or a later version is used. |  |  |

| Rule number description file | This option specifies the file that contains a list of rule numbers. This corresponds to the -misra2004=file options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify a file name. |
|  | Remark | This property can be used when "Apply all rules in the specified file" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Rule number | This option specifies the rule numbers to be checked. Be sure to specify at least one rule number. This corresponds to the -misra2004=apply options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify rule numbers. - Each rule number must be specified using decimal values and a period (.). - To specify two or more rule numbers, separate them with a comma (,). |
|  | Remark | This property can be used when "Apply specified rules" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Exclusion rule number | This option specifies the rule numbers to be excluded from the checking. This corresponds to the -misra2004=ignore options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify rule numbers. - Each rule number must be specified using decimal values and a period (.). - To specify two or more rule numbers, separate them with a comma (,). |
|  | Remark | This property can be used when "Ignore specified rules" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Check rule number besides required rule | This option specifies the rule numbers to be checked in addition to the required rules. This corresponds to the -misra2004=required_add options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify rule numbers. - Each rule number must be specified using decimal values and a period (.). - To specify two or more rule numbers, separate them with a comma (,). |
|  | Remark | This property can be used when "Apply specified required rules" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Exclusion rule number from required rule | This option specifies the rule numbers to be excluded from the required rules. This corresponds to the -misra2004=required_remove options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify rule numbers. - Each rule number must be specified using decimal values and a period (.). - To specify two or more rule numbers, separate them with a comma (,). |
|  | Remark | This property can be used when "Ignore specified required rules" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Output message of the enhanced key word and extended specifications | This option enables the source-code checks of the MISRA-C:2004 rules in the following cases where they are suppressed by the unique language specifications extended from the C language standard. - When the function has no prototype declaration (rule 8.1) and #pragma interrupt is specified for it. Specifying this option in the standard-version compiler will cause an error. This corresponds to the -check_language_extension options of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -check_language_extension | To enable the source-code checking of the MISRA-C:2004 rules, which are partially suppressed by the extended language specifications. |
|  |  | Unchecked | --- | The source-code checking of the MISRA-C:2004 rules is disabled, which are partially suppressed by the extended language specifications. |
|  | Remark | This property can be used when an option other than "Not apply rule" is selected for [Apply rule]. |  |  |

| Rule check exclusion file | This option does not check file against the MISRA-C: 2004 rules. Specifying this option in the standard-version compiler will cause an error. This corresponds to the -ignore_files_misra option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify the file. Use the "Delete", "Edit", "Move Up" or "Move Down" button to delete the selected file or to change its name or position in the list. |
|  | Remark | This property can be used when an option other than "Not apply rule" is selected for [Apply rule]. The existence of the specified file is not checked. When this property field is blank, this option is not output. |

| Apply rule | This option checks source code against the MISRA-C:2012 rules. A message is output if the item specified for the check. Specifying this option in the standard-version compiler will cause an error. This corresponds to the -misra2012 options of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Not apply rule |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Apply all rules | -misra2012=all | The compiler checks the source code against all of the rules that are supported. |
|  |  | Apply specified rules | -misra2012=apply | The compiler checks the source code against the rules with the selected numbers. |
|  |  | Ignore specified rules | -misra2012=ignore | The compiler checks the source code against the rules other than those with the selected numbers. |
|  |  | Apply all required rules | -misra2012=required | The compiler checks the source code against the rules of the "required" type. |
|  |  | Apply specified required rules | -misra2012=required_add | The compiler checks the source code against the rules of the "required" type and the rules with the selected numbers. |
|  |  | Ignore specified required rules | -misra2012=required_remove | The compiler checks the source code against the rules other than those with the selected numbers among the rules of the "required" type. |
|  |  | Apply all rules in the specified file | -misra2012=<file name> | The compiler checks the source code against the rules with the numbers written in the specified file. |
|  |  | Not apply rule | --- | The source code is not checked against the MISRA-C: 2012 rules. |
|  | Remark | This property can be used when CC-RX V2.03.00 or a later version is used. |  |  |

| Rule number description file | This option specifies the file that contains a list of rule numbers. This corresponds to the -misra2012=file options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify a file name. |
|  | Remark | This property can be used when "Apply all rules in the specified file" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Rule number | This option specifies the rule numbers to be checked. Be sure to specify at least one rule number. This corresponds to the -misra2012=apply options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify rule numbers. - Each rule number must be specified using decimal values and a period (.). - To specify two or more rule numbers, separate them with a comma (,). |
|  | Remark | This property can be used when "Apply specified rules" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Exclusion rule number | This option specifies the rule numbers to be excluded from the checking. This corresponds to the -misra2012=ignore options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify rule numbers. - Each rule number must be specified using decimal values and a period (.). - To specify two or more rule numbers, separate them with a comma (,). |
|  | Remark | This property can be used when "Ignore specified rules" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Check rule number besides required rule | This option specifies the rule numbers to be checked in addition to the required rules. This corresponds to the -misra2012=required_add options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify rule numbers. - Each rule number must be specified using decimal values and a period (.). - To specify two or more rule numbers, separate them with a comma (,). |
|  | Remark | This property can be used when "Apply specified required rules" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Exclusion rule number from required rule | This option specifies the rule numbers to be excluded from the required rules. This corresponds to the -misra2012=required_remove options of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify rule numbers. - Each rule number must be specified using decimal values and a period (.). - To specify two or more rule numbers, separate them with a comma (,). |
|  | Remark | This property can be used when "Ignore specified required rules" is selected for [Apply rule]. When this property field is blank, this option is not output. |

| Output message of the enhanced key word and extended specifications | This option enables the source-code checks of the MISRA-C:2012 rules in the following cases where they are suppressed by the unique language specifications extended from the C language standard. - When the function has no prototype declaration (rule 8.1) and #pragma interrupt is specified for it. Specifying this option in the standard-version compiler will cause an error. This corresponds to the -check_language_extension options of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -check_language_extension | To enable the source-code checking of the MISRA-C:2012 rules, which are partially suppressed by the extended language specifications. |
|  |  | Unchecked | --- | The source-code checking of the MISRA-C:2012 rules is disabled, which are partially suppressed by the extended language specifications. |
|  | Remark | This property can be used when an option other than "Not apply rule" is selected for [Apply rule]. |  |  |

| Rule check exclusion file | This option does not check file against the MISRA-C: 2012 rules. Specifying this option in the standard-version compiler will cause an error. This corresponds to the -ignore_files_misra option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify the file. Use the "Delete", "Edit", "Move Up" or "Move Down" button to delete the selected file or to change its name or position in the list. |
|  | Remark | This property can be used when an option other than "Not apply rule" is selected for [Apply rule]. The existence of the specified file is not checked. When this property field is blank, this option is not output. |

- Assembler

- [Description of each category]

- (1) [Source]

- This is used to display detailed information on the options for controlling preprocessor and make changes to the settings.

| Include file directories | This option specifies the name of the path to the folder that stores the include file. A include file is searched for in the order of the current folder, the folders specified by the include option, and the folders specified by environment variable INC_RXA. This corresponds to the -include option of the asrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify the folder. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected folder or to change its name or position in the list. |
|  | Remark | The existence of the specified folder is not checked. |

| Defines | This option replaces the macro name with the specified string. This provides the same function as writing the .DEFINE directive at the beginning of the source file. This corresponds to the -define option of the asrx command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify the user-defined symbol (<name>=<def>). Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected user-defined symbol or to change its name or position in the list. |
|  | Remark | None |

| Input character code | This option specifies the character code to use for strings, character constants and comments. This corresponds to the -sjis, -euc, -latin1, -big5, -gb2312, -utf8 option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | SJIS |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | SJIS | -sjis | Handle the characters in strings, character constants, and comments by using SJIS |
|  |  | EUS | -euc | Handle the characters in strings, character constants, and comments by using EUC |
|  |  | ISO-Latin1 | -latin1 | Handle the characters in strings, character constants, and comments by using ISO-Latin1 |
|  |  | BIG5 | -big5 | Handle the characters in strings, character constants, and comments by using BIG5 |
|  |  | GB2312 | -gb2312 | Handle the characters in strings, character constants, and comments by using GB2312 |
|  |  | UTF-8 | -utf8 | Handle the characters in strings, character constants, and comments by using UTF-8 |
|  | Remark | "BIG5" or "GB2312" can be specified when CC-RX V2.01.00 or a later version is used. "UTF-8" can be specified when CC-RX V2.04.00 or a later version is used. |  |  |

- (2) [Object]

- This is used to display detailed information on the output-related options and make changes to the settings.

| Generate debug information | This option outputs the debugging information to the relocatable file. This corresponds to the -debug, -nodebug option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Depends on the build configuration. - Release build configuration: Unchecked - Other than the release build configuration: Checked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -debug | Output debugging information to relocatable file. |
|  |  | Unchecked | -nodebug | Output no debugging information to relocatable file. |
|  | Remark | None |  |  |

| Inter-module optimization | This option outputs the additional information for the inter-module optimization. At linkage, inter-module optimization is applied to the file specified with this option. This corresponds to the -goptimize option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -goptimize | Output additional information for inter-module optimization. |
|  |  | Unchecked | --- | Output no additional information for inter-module optimization. |
|  | Remark | None |  |  |

| Output no data to unfilled area | When a Motorola S-record file (<name>.mot) or Hex file (<name>.hex) is output, this option blocks spaces created by .OFFSET directives in the assembly language being filled with output data. This corresponds to the -create_unfilled_area option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -create_unfilled_area | Makes spaces created by .OFFSET unfilled. |
|  |  | Unchecked | --- | --- |
|  | Remark | This property can be specified when CC-RX V2.03.00 or a later version is used. |  |  |

| Output file directory | The folder which serves as the origin of the folder where the object file created after assembly will be saved is displayed. A folder having a relative path equivalent to that from the project folder to the folder holding the source file is created in the origin folder. After that, the object file is output to the folder. This corresponds to the -output option of the asrx command. |  |
| --- | --- | --- |
|  | Default | ${CONFIGDIR} |
|  | How to change | Fixed (Read-only) |
|  | Remark | This property option is not displayed in [Assembler] -> [All options]. An object file with a file name made by replacing the extension of the input file name with ".obj". |

| Use floating point arithmetic instructions | This option is used to generate a relocatable file which is capable of containing FPU instructions. This corresponds to the -fpu, -nofpu option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Default |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Default | --- | When "RX200" is selected for [Compiler] -> [CPU] -> [CPU type], it is assumed that "Disable" is specified. The other case is assumed that [Enable] is specified. |
|  |  | Enable | -fpu | Generate a relocatable file which is capable of containing FPU instructions. |
|  |  | Disable | -nofpu | Generate a relocatable file which is not capable of containing FPU instructions |
|  | Remark | This property can be specified when CC-RX V2.01.00 or a later version is used. |  |  |

- (3) [Object]/[Advanced]

- This is reserved for future enhancement.

- (4) [List]

- This is used to display detailed information on the output options for lists and make changes to the settings.

- "Enable all" to enable all list file features.

| Generate list file | This option outputs an assemble list file. This corresponds to the -listfile, -nolistfile option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -listfile="$(basename $(notdir $<)).lst" | Output an assemble list file. |
|  |  | Unchecked | -nolistfile | Output no assemble list file. |
|  | Remark | The assemble list file is output to the folder where the output file is to be output. The name of the assemble list file is the input file name with the extension replaced with ".lst". |  |  |

| (List file contents) False line in conditional assemble | The statements for which the specified condition is not satisfied in conditional assembly are also output to a source list file. This corresponds to the -show option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -show=conditionals | Output the statements unsatisfied in conditional assembly |
|  |  | Unchecked | --- | Output no statements unsatisfied in conditional assembly |
|  | Remark | This property can be used when the check box for [Generate list file] is checked. |  |  |

| (List file contents) Before .DEFINE replacement | The information before replacement specified by .DEFINE is output to a source list file. This corresponds to the -show option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -show=definitions | Output the information before replacement specified with .DEFINE |
|  |  | Unchecked | --- | Output no information before replacement specified with .DEFINE |
|  | Remark | This property can be used when the check box for [Generate list file] is checked. |  |  |

| (List file contents) After macro expansion | The macro expansion statements are output to a source list file. This corresponds to the -show option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -show=expansion | Output the macro expansion statements |
|  |  | Unchecked | --- | Output no macro expansion statements |
|  | Remark | This property can be used when the check box for [Generate list file] is checked. |  |  |

- (5) [Miscellaneous]

- This is used to display detailed information on miscellaneous options for lists and make changes to the settings.

| Checks for a privileged instruction | This option outputs warning W0551011 when a privileged instruction is used in the source file. This corresponds to the -chkpm option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -chkpm | Check for a privileged instruction. |
|  |  | Unchecked | --- | Do not check for a privileged instruction |
|  | Remark | None |  |  |

| Checks for a floating-point operation instruction | This option outputs warning W0551012 when a floating-point operation instruction is used in the source file. This corresponds to the -chkfpu option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -chkfpu | Check for a floating-point operation instruction. |
|  |  | Unchecked | --- | Do not check for a floating-point operation instruction. |
|  | Remark | None |  |  |

| Checks for a DSP instruction | This option outputs warning W0551013 when a DSP instruction is used in the source file. This corresponds to the -chkdsp option of the asrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -chkdsp | Check for a DSP instruction. |
|  |  | Unchecked | --- | Do not check for a DSP instruction. |
|  | Remark | None |  |  |

- (6) [Users]

- This is used to display detailed information on other items regarding the assembly and make changes to the settings.

| User-defined options | Specify compiler options to be added. |  |
| --- | --- | --- |
|  | Default | -nologo |
|  | How to change | Use the "Add" button to specify the optional character string of asrx command. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected option or to change its name or position in the list. |
|  | Remark | The format of the specified option is not checked. The option specified by this property is added as the last of the assembler options group. |

3. Linker

# [Description of each category]

## (1) [Input]

This is used to display detailed information on the input control options and make changes to the settings.

| Relocatable files, object files and library files | Specify the input files to be added. Three types of specification are possible. (1) Object files or modules from the libraries (2) Library files (3) Files of binary data These correspond to the -input, -library, and -binary options of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a file. - Object file: <object file> - Relocatable file: <relocatable file> - Module in library file: <library file>(<module>) - Library file: <library file> - Binary file: <binary file>(<section>[:<alignment>][/<attribute>][,<symbol>]) - <alignment>: 1, 2, 4, 8, 16, or 32 (If alignment is omitted, "1" will be valid by default.) - <attribute>: CODE or DATA (the write, read, and execute attributes will be all valid by default.) Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected file or to change its name or position in the list. |
|  | Remark | The existence of the specified file is not checked. The specified file is automatically added to "Linkage order list". |

## (2) [Input]/[Defines]

This is used to display detailed information on the symbol definition options and make changes to the settings.

| Defines | This option defines an undefined symbol forcedly as an externally defined symbol or a numerical value. This corresponds to the -define option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify an undefined symbol (<symbol name>={<symbol name>¦<numerical value>}). - Specify <value> in hexadecimal. - If the specified value starts with a character from A to F, symbols are searched first, and if corresponding symbol is not found, the value is interpreted as a numerical value. - Values starting with 0 are always interpreted as numerical values. - If the specified symbol name is a C/C++ variable name, add "_" at the head of the definition name in the program. - If the symbol name is a C++ function name (except for the main function), enclose the definition name with the double-quotes including parameter strings. If the parameter is void, specify as "<function name>( )". Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the specified undefined symbol or to change its name or position in the list. |
|  | Remark | None |

## (3) [Input]/[Advanced]

This is used to display detailed information on the settings for the order of linkage and make changes to the settings.

| Prelinker control | This option disables the prelinker initiation. The prelinker supports the functions to generate the C++ template instance automatically and to check types at run time. This corresponds to the -noprelink option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Auto |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Auto | --- | Run the pre-linker if any input file references an Instance Information File (*.ii) |
|  |  | Skip prelinker | -noprelink | Disable the prelinker initiation. Reduce the link time. |
|  |  | Run prelinker | --- | Enable the prelinker initiation. |
|  | Remark | None |  |  |

| Use entry point | This option specifies the execution start address with an externally defined symbol or address. This corresponds to the -entry option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -entry | Specify execution start address - The address is specified in hexadecimal notation (e.g. A0FFF - NB: do not precede with 0x). - If the specified value starts with a letter from A to F, symbols are searched first, and if no corresponding symbol is found, the value is interpreted as an address. Values starting with 0 are always interpreted as addresses. - For a C function name, add an underscore (_) at the head of the definition name in the program. - For a C++ function name (except for the main function), enclose the definition name with double-quotes in the program including parameter strings. - If the parameter is void, specify as "<function name>()". |
|  |  | Unchecked | --- | Specify no execution start address |
|  | Remark | None |  |  |

| Use custom linkage order | This allows customizing the order of linkage of input files. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | --- | Use the list of the order of linkage to specify the order of linking the input files. |
|  |  | Unchecked | --- | Use no list of the order of linkage. The input files are linked in the alphabetical order. |
|  | Remark | None |  |  |

| (Linkage order list) | Specify the linkage order of input files. |  |
| --- | --- | --- |
|  | Default | Alphabetical order |
|  | How to change | Use the [Move Up] and [Move Down] buttons to operate the files in the list. |
|  | Remark | Object files which are the results of compiling or assembling the source files registered in a project and the files specified for [Relocatable files, object files and library files] are automatically registered in the linkage order list. |

## (4) [List]

This is used to display detailed information on the output options for lists and make changes to the settings.

| Generate list file | This option outputs a list file. This corresponds to the -list option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Checked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -list=${CONFIGDIR:/${ArtifactName}}.map | Output a linker map file |
|  |  | Unchecked | --- | Output no linker map file |
|  | Remark | The linkage map file is output to the folder where the output file is to be output. The name of the linkage map file is the output file name with the extension replaced with ".map". |  |  |

| Output method | This option specifies output contents of a list. This corresponds to the -show option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Default |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | All | -show=all | Output the same information as when the check boxes for [Show symbol], [Show reference], [Show cross-reference], and [Show total section-size] are checked. |
|  |  | Default | -show | Output the same information as when the check boxes for [Show symbol] and [Show reference] are checked. |
|  |  | Custom | -show=<info> | The specified output information is output to the linkage map file. |
|  | Remark | This property can be used when the check box for [Generate list file] is checked. |  |  |

| Show symbol | This option outputs symbol address, size, type, and optimization contents. This option outputs a symbol name list in a module when [System Library] or [User Library] is selected for [Output] -> [Type of output file]. This corresponds to the -show=symbol option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -show=symbol | Output a symbol information to the list file |
|  |  | Unchecked | --- | Output no symbol information to the list file |
|  | Remark | This property can be used when "Custom" is selected for [Output method]. |  |  |

| Show reference | This option outputs the number of symbol references. This corresponds to the -show=reference option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -show=reference | Output the number of symbol references to the list file |
|  |  | Unchecked | --- | Output no number of symbol references to the list file |
|  | Remark | This property can be used when "Custom" is selected for [Output method]. This property cannot be used when "System Library" or "User Library" or "Relocatable" is selected for [Output] -> [Type of output file]. |  |  |

| Show cross-reference | This option outputs the cross-reference information. This corresponds to the -show=xreference option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -show=xreference | Output the cross reference information to the list file |
|  |  | Unchecked | --- | Output no cross reference information to the list file |
|  | Remark | This property can be used when "Custom" is selected for [Output method]. This property cannot be used when "System Library" or "User Library" is selected for [Output] -> [Type of output file]. |  |  |

| Show total section-size | This option outputs the total sizes of sections allocated to the ROM and RAM areas. This corresponds to the -show=total_size option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -show=total_size | Show the total sizes of sections allocated to the ROM and RAM areas |
|  |  | Unchecked | --- | Show no total sizes of sections |
|  | Remark | This property can be used when "Custom" is selected for [Output method]. This property cannot be used when "System Library" or "User Library" is selected for [Output] -> [Type of output file]. |  |  |

| Show vector | This option outputs vector information. This corresponds to the -show=vector option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -show=vector | Output vector information to the list file |
|  |  | Unchecked | --- | Output no vector information to the list file |
|  | Remark | This property can be used when "Custom" is selected for [Output method]. This property cannot be used when "System Library" or "User Library" or "Relocatable" is selected for [Output] -> [Type of output file]. |  |  |

| Show section | This option outputs a section list in a module. This corresponds to the -show=section option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -show=section | Output a section list in a module. |
|  |  | Unchecked | --- | Output no section list in a module. |
|  | Remark | This property can be used when "Custom" is selected for [Output method]. This property can be used when "System Library" or "User Library" is selected for [Output] -> [Type of output file]. |  |  |

## (5) [Optimize]

This is used to display detailed information on the optimization options and make changes to the settings.

| Optimize | When this option is specified, optimization is performed for the file specified with the goptimize option at compilation or assembly. This corresponds to the -optimize and -nooptimize options of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | None |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | All | -optimize | Execute all optimization. It has the same meaning as when the check boxes for [Eliminate dead code], [Eliminate same code], [Use short disp/imm], and [Optimize branches] are checked. |
|  |  | Speed | -optimize=speed | Execute optimizations other than those reducing object speed. It has the same meaning as when the check boxes for [Eliminate dead code], [Use short disp/imm], and [Optimize branches] are checked. |
|  |  | Safe | -optimize=safe | Execute optimization other than those limited by variable or function attributes. It has the same meaning as when the check boxes for [Use short disp/imm] and [Optimize branches] are checked. |
|  |  | Custom | --- | Perform optimization for the specified options. |
|  |  | None | -nooptimize | Optimization is not performed at linkage. |
|  | Remark | This property cannot be used when "System Library" or "User Library" is selected for [Output] -> [Type of output file]. |  |  |

| Optimization enabled Eliminate dead code | This option deletes variables and functions that are never referenced. This corresponds to the -optimize=symbol_delete option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -optimize=symbol_delete | Delete variables and functions that are not referenced. |
|  |  | Unchecked | --- | Delete no variables and functions that are not referenced. |
|  | Remark | This property can be used when "Custom" is selected for [Optimize]. |  |  |

| Optimization enabled Eliminate same code | This option creates a subroutine for the same instruction sequence. This corresponds to the -optimize=same_code option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -optimize=same_code | Create a subroutine for the same instruction sequence. |
|  |  | Unchecked | --- | Create no subroutine for the same instruction sequence. |
|  | Remark | This property can be used when "Custom" is selected for [Optimize]. |  |  |

| Optimization enabled Optimize branches | This option optimizes branch instruction size according to program allocation information. Even if this option is not specified, it is performed when any other optimization is executed. This corresponds to the -optimize=branch option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -optimize=branch | Optimize branch instruction size according to program allocation information. |
|  |  | Unchecked | --- | Optimize no branch instruction size. |
|  | Remark | This property can be used when "Custom" is selected for [Optimize]. |  |  |

| Optimization enabled Use short disp/imm | This option replaces an instruction having a displacement or an immediate value with a smaller-size instruction when the code size of the displacement or immediate value can be reduced. This corresponds to the -optimize=short_format option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -optimize=short_format | Replace an instruction with a smaller-size instruction. |
|  |  | Unchecked | --- | Replace no instruction with a smaller-size instruction. |
|  | Remark | This property can be used when "Custom" is selected for [Optimize]. |  |  |

| Minimum code size | This option specifies the minimum code size for the optimization with [Eliminate same code]. This corresponds to the -samesize option of the rlink command. |  |
| --- | --- | --- |
|  | Default | 1E (hexadecimal number) |
|  | How to change | Specify a value in the range from 0x8 and 0x7FFF in hexadecimal notation. |
|  | Remark | This property cannot be used when "System Library" or "User Library" is selected for [Output] -> [Type of output file]. This property can be used when either of the following conditions is met. - The check box for [Eliminate same code] is checked. - "All" is selected for [Optimize]. |

| Unreferenced symbol that disables deletion by optimization | This option disables optimization regarding unreferenced symbol deletion. This corresponds to the -symbol_forbid option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a symbol (<symbol>). - For a C/C++ variable or C function name, add an underscore (_) at the head of the definition name in the program. - For a C++ function, enclose the definition name in the program with double-quotes including the parameter strings. - When the parameter is void, specify as "<function name>( )". Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the specified symbol or to change its name or position in the list. |
|  | Remark | This property cannot be used when "System Library" or "User Library" is selected for [Output] -> [Type of output file]. This property cannot be used when "None" is selected for [Optimize]. |

| Same-code that disables unification regarding optimization | This option disables optimization regarding same-code unification. This corresponds to the -samecode_forbid option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a function (<function>). - For a C/C++ variable or C function name, add an underscore (_) at the head of the definition name in the program. - For a C++ function, enclose the definition name in the program with double-quotes including the parameter strings. - When the parameter is void, specify as "<function name>( )". Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected function or to change its name or position in the list. |
|  | Remark | This property cannot be used when "None" is selected for [Optimize]. |

| Section to disable optimization | This option disables optimization for the specified section. If an input file name or library module name is also specified, the optimization can be disabled for a specific file, not only the entire section. This corresponds to the -section_forbid option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a section ([<file name>¦<module name>](<section name>)). Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected section or to change its name or position in the list. |
|  | Remark | This property cannot be used when "System Library" or "User Library" is selected for [Output] -> [Type of output file]. This property cannot be used when "None" is selected for [Optimize]. |

| Address range to disable optimization | This option disables optimization regarding address + size specification. This corresponds to the -absolute_forbid option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify an address range (<address> [+<size>]). Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected address range or to change its name or position in the list. |
|  | Remark | This property cannot be used when "System Library" or "User Library" is selected for [Output] -> [Type of output file]. This property cannot be used when "None" is selected for [Optimize]. |

## (6) [Optimize]/[Advanced]

This is used to display detailed information on the inter-module optimization options and make changes to the settings.

| Generate file for inter-module optimization of external variable accesses (-map) | This option outputs the external-symbol-allocation information file that is used by the compiler in optimizing access to external variables. When this option is specified and "Inner-module" is specified in [Compiler] -> [Optimize] -> [Advanced] -> [Optimization for access to external variables], an application is built twice. In the first build processing, the linker generates a ".bls" file that includes data related to all external variable access in the application. In the second build processing, the compiler uses the generated file as an input and optimizes the external variable access. This corresponds to the -map option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -map | Divide load module file |
|  |  | Unchecked | --- | Do not divide load module file |
|  | Remark | This property cannot be used when "System Library" or "User Library" is selected for [Output] -> [Type of output file]. |  |  |

## (7) [Section]

This is used to display detailed information on option for specifying sections and make changes to the settings.

| Sections viewer | This option specifies the start address of the section. This corresponds to the -start option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Depend on target device |
|  | How to change | Use "Add section" to specify the sections and address. Use the "Remove the section", "Move Up", or "Move Down" button to delete the selected section or to change its position in the list. |
|  | Remark | None |

| Override Linker Script | This option specifies a linker script file. Override current linker section settings with the new settings. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked Blank |  |  |
|  | How to change | Select or deselect the check box. Specify a file to open in the [linker script file] editbox. - Use "Browse" button to select the linker script (*.xmi.fragment). |  |  |
|  | Explanation in detail | Checked | --- | Produces the [linker script file] editbox. |
|  |  | Unchecked | --- | Use no linker script files. |
|  | Remark | "Import" button - Save the settings for the current section in the specified file. "Export" button - Reflect the contents of the specified file in the settings for the current section. "Re-Apply" button - When the check box for [Override linker script] is not checked and the [linker script file] editbox is blank, information on the section is returned to the state when a project is newly created. - When the check box for [Override linker script] is checked and the [linker script file] editbox is blank, no operation is performed. - When the check box for [Override linker script] is checked and a file is selected for [linker script file] editbox, the information on the section is overwritten with the contents of the file. |  |  |

## (8) [Section]/[Symbol file]

This is used to display detailed information on the external definition symbol file options and make changes to the settings.

| Symbol file | This option outputs externally defined symbols in the specified section to a file in the assembler directive format. The file name is <output file>.fsy. This corresponds to the -fsymbol option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a section. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected file or to change its name or position in the list. |
|  | Remark | None |

## (9) [Section]/[Advanced]

This is used to display detailed information on the external definition symbol file options and make changes to the settings.

| Checks the section larger than the specified range of address | A section larger than the specified range of addresses leads to an error. This corresponds to the -cpu option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box. |  |  |
|  | Explanation in detail | Checked | -cpu | Enable [Address range of the memory type]. |
|  |  | Unchecked | --- | Disable [Address range of the memory type]. |
|  | Remark | This property cannot be used when "Relocatable", "System Library" or "User Library" is selected for [Output] -> [Type of output file]. |  |  |

| Address range of the memory type | This option specifies the memory type and its address range of which consistency is to be checked. This corresponds to the -cpu option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a memory type and its address range (<type>=<address1>-<address2>). - The items that can be specified as <type> are shown below. - ROm: Allocates the section to a ROM area. - RAm: Allocates the section to a RAM area. - FIX: Allocates the section to a fixed-address area (e.g. I/O area). - Specify the start address and end address of the address range to check for consistency in hexadecimal as <address1> and <address2>. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the specified section allocation address or to change its name or position in the list. |
|  | Remark | This option can be used when the check box for [Check section larger then specified range of address] is checked. When this property field is blank, this option is not output. |

| Allocates to the next area of the same memory type or the section is divided | A section larger than the specified range of addresses is allocated to the next area of the same memory type or the section is divided. This corresponds to the -cpu option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box. |  |  |
|  | Explanation in detail | Checked | -cpu=stride | A linkage error occurs when they are not allocated within the ROM area even though the section is divided. |
|  |  | Unchecked | --- | An error is output when a section is not allocated within the specified address range. |
|  | Remark | This option can be used when at least one address range is selected for [Address range of the memory type]. |  |  |

| Not divide the specified section | Allocates the specified section to another available area of the same memory type without dividing the section. This corresponds to the -contiguous_section option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a section name. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the specified section name or to change its name or position in the list. |
|  | Remark | This option can be used when the check box for [Allocates to the next area of the same memory type or the section is divided] is checked. When this property field is blank, this option is not output. |

| Section alignment | Changes the alignment value for the specified section to 16 bytes. This corresponds to the -aligned_section option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a section name. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the specified section name or to change its name or position in the list. |
|  | Remark | This property cannot be used when "Relocatable", "System Library" or "User Library" is selected for [Output] -> [Type of output file]. When this property field is blank, this option is not output. |

## (10) [User]

This is used to display detailed information on other items regarding the linker and make changes to the settings.

| User-defined options | Specify linker options to be added. |  |
| --- | --- | --- |
|  | Default | -nomessage -nologo |
|  | How to change | Use the "Add" button to specify an optional character string of rlink command. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected option or to change its name or position in the list. |
|  | Remark | The format of the specified option is not checked. The option specified by this property is added as the last of the linker options group. |

## (11) [Subcommand file]

This is used to display detailed information on the subcommand file options and make changes to the settings.

| Use external sub-command file | This option specifies options with a subcommand file. Option contents specified with a subcommand file are expanded to the location at which the subcommand is specified on the command line and are executed. This corresponds to the -subcommand option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -subcommand=<file name> | Specify options with a subcommand file. |
|  |  | Unchecked | --- | Specify no options with a subcommand file. |
|  | Remark | None |  |  |

| Sub-command file path | This option specifies options with a subcommand file. This corresponds to the -subcommand option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Browse" button to specify a linker command file. - The format of the subcommand file is as follows: <option> { = ¦  } [<suboption> [,E ] [ &] [;<comment>] - The option and suboption are separated by an "=" sign or a space. - For the input option, suboptions are separated by a space. - One option is specified per line in the subcommand file. - If a subcommand description exceeds one line, the description can be allowed to overflow to the next line by using an ampersand (&). - The subcommand option cannot be specified in the subcommand file. |
|  | Remark | This property can be used when the check box for [Use external sub-command file] is checked. |

## (12) [Output]

This is used to display detailed information on the output control options and make changes to the settings.

| Debug information | This option outputs information on debugging to an output file. This corresponds to the -nodebug, -debug, -sdebug option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Depends on the build configuration - Release build configuration: None - Other than the release build configuration: In output load module |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | None | -nodebug | Do not output a debug information. |
|  |  | In output load module | -debug | Output a debug information to the output file. |
|  |  | In separate debug file (*.dbg) | -sdebug | Output a debug information to <output file name>.dbg file |
|  | Remark | None |  |  |

| Data record header | This option outputs data with the specified data record regardless of the address range. If there is an address that is larger than the specified data record, the appropriate data record is selected for the address. This corresponds to the -record option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | None |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | None | --- | --- |
|  |  | S1 | -record=S1 | Use 16-bit in the generated S-Record file |
|  |  | S2 | -record=S2 | Use 24-bit in the generated S-Record file |
|  |  | S3 | -record=S3 | Use 32-bit in the generated S-Record file |
|  | Remark | This property can be used when "Stype via absolute" is selected for [Type of output file]. |  |  |

| Output file path | This option specifies an output file name. The extension of the output file name automatically changes when [Type of output file] is modified. This corresponds to the -output option of the rlink command. |  |
| --- | --- | --- |
|  | Default | - When "Absolute (ELF)" is selected for the [Type of output file]: ${CONFIGDIR:/${ProjName}}.abs - When "Relocatable" is selected for the [Type of output file]: ${CONFIGDIR:/${ProjName}}.rel - When "System library" is selected for the [Type of output file]: ${CONFIGDIR:/${ProjName}}.lib - When "User library" is selected for the [Type of output file]: ${CONFIGDIR:/${ProjName}}.lib - When "Hex via absolute" is selected for the [Type of output file]: ${CONFIGDIR:/${ProjName}}.hex - When "Stype via absolute" is selected for the [Type of output file]: ${CONFIGDIR:/${ProjName}}.mot - When "Binary via absolute" is selected for the [Type of output file]: ${CONFIGDIR:/${ProjName}}.bin |
|  | How to change | Fixed (Read-only) |
|  | Remark | To change the name of the output file, change the name in [Artifact Name] under the [Build Artifact] tab. |

| Type of output file | This option specifies the output format. This corresponds to the -form option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Stype via absolute (for application project) |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Absolute (ELF) | -form=absolute | Output an absolute file (ELF) |
|  |  | Absolute (SYSROF) | --- | Absolute (SYSROF) format is not supported for RXC toolchain. |
|  |  | Relocatable | -form=relocate | Output a relocatable file |
|  |  | System library | -form=library=s | Output a system library file |
|  |  | User library | -form=library=u | Output a user library file |
|  |  | Hex via absolute | -form=hexadecimal | Output a HEX file |
|  |  | Stype via absolute | -form=stype | Output an S-type file |
|  |  | Binary via absolute | -form=binary | Output a binary file |
|  | Remark | None |  |  |

| Output no data to unfilled area | When a Motorola S-record file or Hex file is output, this option blocks spaces created by .OFFSET directives in the assembly language being filled with output data. This corresponds to the -create_unfilled_area option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -create_unfilled_area | Output no data to unfilled areas. |
|  |  | Unchecked | --- | --- |
|  | Remark | This property can be specified when CC-RX V2.03.00 or a later version is used. |  |  |

| Specifies byte count for data record | Specifies the maximum byte count for a data record when a file is to be created in the Intel-Hex format. This corresponds to the -byte_count option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -byte_count | Specifies the maximum number of bytes in a data record. |
|  |  | Unchecked | --- | --- |
|  | Remark | This property can be used when "Hex via absolute" is selected for [Type of output file]. |  |  |

| Maximum byte count for data record | This option specifies the maximum byte count for a data record when a file is to be created in the Intel-Hex format. This corresponds to the -byte_count option of the rlink command. |  |
| --- | --- | --- |
|  | Default | FF (hexadecimal number) |
|  | How to change | Specify a value in the range from 0x0 and 0xFF in hexadecimal notation. |
|  | Remark | This property can be used when the check box for [Specifies byte count for data record] is checked. |

| Full unused areas in the output ranges with the value | This option fills the unused areas in the output ranges with random values or a user-specified hexadecimal value. The following unused areas are filled with the value according to the output range specification in the [Division output file]: - When section names are specified for the output range: The specified value is output to unused areas between the specified sections. - When an address range is specified for the output range: The specified value is output to unused areas within the specified address range. A 1-, 2-, or 4-byte value can be specified. The hexadecimal value specified to the space option determines the output data size. If a 3-byte value is specified, the upper digit is extended with 0 to use it as a 4-byte value. If an odd number of digits are specified, the upper digits are extended with 0 to use it as an even number of digits. If the size of an unused area is not a multiple of the size of the specified value, the value is output as many times as possible, then a warning message is output. This corresponds to the -space option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes(Random) | -space=random | Fill the areas with random values. |
|  |  | Yes(Specification value) | -space=< numerical value> | Fill the areas with a hexadecimal value. |
|  |  | No | --- | Do not fill the areas. |
|  | Remark | This property can be used when "Division output file" is not left blank. |  |  |

| Output padding data | This option specifies data used to fill the unused areas. This corresponds to the -space option of the rlink command. |  |
| --- | --- | --- |
|  | Default | FF (hexadecimal number) |
|  | How to change | Specify a value in the range from 0x0 and 0xFFFFFFFF in hexadecimal notation. |
|  | Remark | This property can be used when "Yes(Specification value)" is selected for [Full unused areas in the output ranges with the value]. |

| Division output file | This option outputs a hex file for the specified address range. This corresponds to the -output option of the linker. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify the output file to be split (<file name>={<start address>-<end address>¦<section name>[:<section name>]...}). - An address is specified in the hexadecimal notation. - If the specified data starts with a letter from A to F, sections are searched first, and if no corresponding section is found, the data is interpreted as an address. - Data starting with 0 are always interpreted as addresses. Use the "Delete", "Edit", "Move Up", or "move Down" button to delete the selected split output file or to change its name or position in the list. |
|  | Remark | This property can be used when "Hex via absolute" or "Stype via absolute" or "Binary via absolute" is selected for [Type of output file]. |

| ROM to RAM mapped section | This option reserves ROM and RAM areas in the initialized data area and relocates a defined symbol in the ROM section with the specified address in the RAM section. This corresponds to the -rom option of the rlink command. |  |
| --- | --- | --- |
|  | Default | D=R D_1=R_1 D_2=R_2 |
|  | How to change | Use the "Add" button to specify a section (<ROM section>=<RAM section>). - ROM section: Specify a relocatable section with initial values. - RAM section: Specify a nonexistent section or a relocatable section whose size is zero. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected file or to change its name or position in the list. |
|  | Remark | This property cannot be used when "Relocatable" or "System library" or "User library" is selected for [Type of output file]. |

## (13) [Output]/[Advanced]

This is used to display detailed information on the other output control options and make changes to the settings.

| Output a stack consumption information file | This option outputs a stack consumption information file. The output file name is ${CONFIGDIR:/${ArtifactName}}.sni. This corresponds to the -stack option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -stack | Output a stack consumption information file. |
|  |  | Unchecked | --- | Output no stack consumption information file. |
|  | Remark | This property cannot be used when "Relocatable" or "System library" or "User library" is selected for [Type of output file]. |  |  |

| Fills in padding data at the end of a section | Fills in padding data at the end of a section so that the section size is a multiple of the boundary alignment of the section. The value of the created padding data is 0x00. Since padding is not performed to an absolute address section, the size of an absolute address section should be adjusted by the user. This corresponds to the -padding option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -padding | Fill in padding data at the end of a section. |
|  |  | Unchecked | --- | Fill in no padding data at the end of a section. |
|  | Remark | None |  |  |

| Output S9 record at the end | This option outputs the S9 record at the end even if the entry address exceeds 0x10000. This corresponds to the -s9 option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -s9 | Output the S9 record at the end. |
|  |  | Unchecked | --- | Output no S9 record at the end. |
|  | Remark | This property can be used when "System library" is selected for [Output] -> [Type of output file] |  |  |

| Compresses the debugging information | The debugging information is compressed. This corresponds to the -compress, -nocompress option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -compress | The debugging information is compressed. |
|  |  | Unchecked | -nocompress | The debugging information is not compressed. |
|  | Remark | This property can be used when an option other than "None" is selected for [Output] -> [Debug information]. |  |  |

| Reduces the memory occupancy | This option specifies the memory size occupied for linkage. This corresponds to the -memory option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -memory=low | The linkage editor loads the information necessary for linkage in smaller units to reduce the memory occupancy. This increases file accesses and processing becomes slower when the occupied memory size is less than the available memory capacity. |
|  |  | Unchecked | -memory=high | The processing is the same as usual. |
|  | Remark | This property can be used when the following conditions are met. - [Compresses the debugging information] is not checked. - "None" is selected for [Optimize]. - [Output a stack consumption information file] is not checked. - Any of the following is specified. - [List] -> [Generate list file] is not checked. - [List] -> [Generate list file] is checked and "Default" is selected for [Output method]. - [List] -> [Generate list file] is checked, "Custom" is selected for [Output method], and Show reference] and [Show cross-reference] are not checked. |  |  |

| Deletes local symbol name information | Deletes local symbol name information from the output file. Since all the name information regarding local symbols is deleted, local symbol names cannot be checked even if the file is opened with a binary editor. This option does not affect the operation of the generated file. The following types of symbol names are hidden: - C source: Variable or function names specified with the static qualifiers - C source: Label names for the goto statements - Assembly source: Symbol names of which external definition (reference) symbols are not declared The entry function name is not hidden. This corresponds to the -hide option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -hide | Delete local symbol name information. |
|  |  | Unchecked | --- | Delete no local symbol name information. |
|  | Remark | None |  |  |

| Select whether to generate CRC code | (CC-RX V2.03.00 or an earlier version is used.) This option is used for cyclic redundancy checking (CRC) of values from the lowest to the highest address of each target range and outputs the calculation result to the specified address. The endian can be specified only when the CPU type is RX Family. When the endian is specified, the calculation result is output to the specified address in the specified endian. When the endian is not specified, the result is output to the specified address in the endian used in the absolute file. This corresponds to the -crc option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes (CRC-CCITT, Automatic endian) | -crc=<OutputAddress>=<TargetRange1>[,<TargetRange2>,E | Generate CRC-CCITT code with data byte arrangement in big endian or in little endian. The endian value is automatically detected. |
|  |  | Yes (CRC-CCITT, Big-endian data) | -crc=<OutputAddress>=<TargetRange1>[,<TargetRange2>,E:big | Generate CRC-CCITT code with data byte arrangement in big endian. |
|  |  | Yes (CRC-CCITT, Little-endian data) | -crc=<OutputAddress>=<TargetRange1>[,<TargetRange2>,E:little | Generate CRC-CCITT code with data byte arrangement in little endian. |
|  |  | Yes (CRC-16, Automatic endian) | -crc=<OutputAddress>=<TargetRange1>[,<TargetRange2>,E/16 | Generate CRC-16 code with data byte arrangement in big endian or in little endian. The endian value is automatically detected. |
|  |  | Yes (CRC-16, Big-endian data ) | -crc=<OutputAddress>=<TargetRange1>[,<TargetRange2>,E/16:big | Generate CRC-16 code with data byte arrangement in big endian. |
|  |  | Yes (CRC-16, Little-endian data) | -crc=<OutputAddress>=<TargetRange1>[,<TargetRange2>,E/16:little | Generate CRC-16 code with data byte arrangement in little endian. |
|  |  | No | --- | Do not generate CRC code. |
|  | Remark | This property can be used when "Hex via absolute" or "Stype via absolute" is selected for [Type of output file]. |  |  |

| Output address | (CC-RX V2.03.00 or an earlier version is used.) Specify the output address. This corresponds to the -crc option of the rlink command. |  |
| --- | --- | --- |
|  | Default | 00000000 |
|  | How to change | Specify a value in the range from 0x0 and 0xFFFFF in hexadecimal notation. |
|  | Remark | This property can be used when "No" is not selected for [Select whether to generate CRC code]. |

| Target range | (CC-RX V2.03.00 or an earlier version is used.) Specify the target rang. This corresponds to the -crc option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify the address range (<start address>-<end address>). - Specify a value in the range from 0x0 and 0xFFFFF in hexadecimal notation. Use the "Delete", "Edit", "Move Up", or "move Down" button to delete the selected the address range or to change its name or position in the list. |
|  | Remark | This property can be used when "No" is not selected for [Select whether to generate CRC code]. |

| Outputs the calculation result of CRC | (CC-RX V2.04.00 or a later version is used.) CRC (cyclic redundancy check) operation is done for the specified range of load objects in the order from the lower to the higher addresses, and the result is output to the specified output address in the specified endian. This corresponds to the -crc option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -crc | The CRC operation and outputting the result are performed. |
|  |  | Unchecked | --- | The CRC operation and outputting the result are not performed. |
|  | Remark | This property can be used when "Hex via absolute" or "Stype via absolute" is selected for [Type of output file]. |  |  |

| Output address | (CC-RX V2.04.00 or a later version is used.) Specify the output address. This corresponds to the -crc option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify a value in the range from 0x0 and 0xFFFFF in hexadecimal notation. |
|  | Remark | This property can be used when the check box for [Outputs the calculation result of CRC] is checked. |

| Type of CRC | (CC-RX V2.04.00 or a later version is used.) Specify the CRC (cyclic redundancy check) operation method. This corresponds to the -crc option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | CRC-CCITT(MSB) type |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | CCITT type | --- | Output the calculation result of CRC-16-CCITT-MSB first operation with an initial value of 0xffff and inverse of XOR. |
|  |  | CRC-CCITT(MSB) type | --- | Output the calculation result of CRC-16-CCITT-MSB first operation. |
|  |  | CRC-CCITT(MSB, LITTLE, 4 byte) type | --- | Output the calculation result of CRC-16-CCITT-MSB first operation with the input specified as 4-byte units in little-endian mode. |
|  |  | CRC-CCITT(MSB, LITTLE, 2 byte) type | --- | Output the calculation result of CRC-16-CCITT-MSB first operation with the input specified as 2-byte units in little-endian mode. |
|  |  | CRC-CCITT(LSB) type | --- | Output the calculation result of CRC-16-CCITT-LSB first operation. |
|  |  | 16 | --- | Output the calculation result of CRC-16-LSB first operation. |
|  |  | SENT(MSB) type | --- | Output the calculation result of operation confirming to SENT. |
|  |  | 32-ETHERNET type | --- | Output the calculation result of CRC-32-ETHERNET operation. |
|  | Remark | This property can be used when the check box for [Outputs the calculation result of CRC] is checked. |  |  |

| Initial value | (CC-RX V2.04.00 or a later version is used.) Specify the initial value. This corresponds to the -crc option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify a following value in hexadecimal notation. - When other than [32-ETHERNET type] is selected in the [Type of CRC]. 0 to FFFF (hexadecimal number) - When [32-ETHERNET type] is selected in the [Type of CRC]. 0 to FFFFFFFF (hexadecimal number) |
|  | Remark | This property can be used when the check box for [Outputs the calculation result of CRC] is checked. |

| Endian and Output size: | (CC-RX V2.04.00 or a later version is used.) Specify the endian, size and offset. This corresponds to the -crc option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | LITTLE-2-0 |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | LITTLE-2-0 | LITTLE-2-0 | Little endian, size is 2, offset is 0. |
|  |  | LITTLE-4-0 | LITTLE-4-0 | Little endian, size is 4, offset is 0. |
|  |  | BIG-2-0 | BIG-2-0 | Big endian, size is 2, offset is 0. |
|  |  | BIG-4-0 | BIG-4-0 | Big endian, size is 4, offset is 0. |
|  | Remark | This property can be used when the check box for [Outputs the calculation result of CRC] is checked. |  |  |

| Target range | (CC-RX V2.04.00 or a later version is used.) Specify the address range. This corresponds to the -crc option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify the address range (<start address>-<end address>). - Specify a value in the range from 0x0 and 0xFFFFF in hexadecimal notation. Use the "Delete", "Edit", "Move Up", or "move Down" button to delete the selected the address range or to change its name or position in the list. |
|  | Remark | This property can be used when the check box for [Outputs the calculation result of CRC] is checked. |

| Address setting for specified area of vector table | This option assigns the specified address to the specified vector number in the variable vector table section. A variable vector table section is created and the specified address is set in the table even if there is no interrupt function in the source code. This corresponds to the -vectn option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a vector information (<vector number>={<symbol>¦<address>}). - Specify a decimal value from 0 to 255 for <vector number>. - Specify the external name of the target function for <symbol>. - Specify the desired hexadecimal address for <address>. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected vector information or to change its name or position in the list. |
|  | Remark | None |

| Address setting for unused vector area | This option assigns the specified address to the vector number to which no address has been assigned in the variable vector table section. A variable vector table section is created by the linkage editor and the specified address is set in the table even if there is no interrupt function in the source code. This corresponds to the -vect option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify a symbol or an address ({<symbol>¦<address>)). - Specify the external name of the target function for <symbol>. - Specify the desired hexadecimal address for <address>. |
|  | Remark | None |

| Outputs the jump table | This option outputs an assembly-language source for a jump table to branch to external definition symbols in the specified section. This corresponds to the -jump_entries_for_pic option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -jump_entries_for_pic | Output an assembly-language source for a jump table. |
|  |  | Unchecked | --- | Output no assembly-language source for a jump table. |
|  | Remark | None |  |  |

| The section that outputs a jump table to branch to external definition symbols | Specify the section. This corresponds to the -jump_entries_for_pic option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the "Add" button to specify a section. Use the "Delete", "Edit", "Move Up", or "Move Down" button to delete the selected section or to change its name or position in the list. |
|  | Remark | This property can be used when the check box for [Outputs the jump table] is checked. |

## (14) [Miscellaneous]

This is used to display detailed information on the other output control options and make changes to the settings.

| Enable information-level message output | This option outputs the information-level messages. This corresponds to the -message or -nomessage option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -message | Output information-level messages. |
|  |  | Unchecked | -nomessage | Suppress the output of information-level messages. |
|  | Remark | This property can be used when the check box for [Use same message related settings as Linker page] is not checked. |  |  |

| Suppress number of information-level message | When an error number is specified, the output of the specified information-level message will be disabled. If a warning-level or error-level message number is specified, the message output is disabled assuming that change_message has changed the specified message to an information-level message. This corresponds to the -nomessage option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify the error number. - Multiple error numbers can be specified by separating them with a comma (,). - A range or error numbers can be specified using a hyphen (-). - When this property is blank, this option is not output. - Specify the 4-digit number that is output after the component number (05) and the phase of occurrence (6) as the error number (for example, specify 0004 for error number M0560004). - 0s at the beginning of the 4-digit number can be omitted (for example, specify 4 for error number M0560004). |
|  | Remark | This property can be used when the check box for [Enable information-level message output] is not checked. |

| Change warning-level message to information-level message | This option changes warning-level messages to information-level messages. This corresponds to the -change_message option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes(All) | -change_message=information | All warning-level messages are changed to information-level messages. |
|  |  | Yes(Specify message number) | -change_message=information=[num] | The warning-level messages with the specified error numbers are changed to information-level messages. |
|  |  | No | --- | Do not change the message level. |
|  | Remark | This property can be used when the check box for [Use same message related settings as Linker page] is not checked. |  |  |

| Number of warning-level message | Specify the error number for the warning-level message to be changed to an information-level message. This corresponds to the -change_message option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify the error number. - Multiple error numbers can be specified by separating them with a comma (,). - A range of error numbers can be specified using a hyphen (-). - Specify the 4-digit number that is output after the component number (05) and the phase of occurrence (6) as the error number (for example, specify 2310 for error number E0562310). - When this property is blank, this option is not output. |
|  | Remark | This property can be used when "Yes(Specify message number)" is selected for [Change warning-level message to information-level message]. |

| Change error-level message to warning-level message | This option changes error-level messages to warning-level messages. This corresponds to the -change_message option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes(All) | -change_message=warning | All error-level messages are changed to warning-level messages. |
|  |  | Yes(Specify message number) | -change_message=warning=[num] | The error-level messages with the specified error numbers are changed to warning-level messages. |
|  |  | No | --- | Do not change the message level. |
|  | Remark | This property can be used when the check box for [Use same message related settings as Linker page] is not checked. |  |  |

| Number of error-level message | Specify the error number for the error-level message to be changed to a warning-level message. This corresponds to the -change_message option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify the error number. - Multiple error numbers can be specified by separating them with a comma (,). - A range of error numbers can be specified using a hyphen (-). - Specify the 4-digit number that is output after the component number (05) and the phase of occurrence (6) as the error number (for example, specify 2310 for error number E0562310). - When this property is blank, this option is not output. |
|  | Remark | This property can be used when "Yes(Specify message number)" is selected for [Change error-level message to warning-level message]. |

| Change information-level and warning-level message to error-level message | This option changes information-level and warning-level messages to error-level messages. This corresponds to the -change_message option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes(All) | -change_message=error | All information-level and warning-level messages are changed to error-level messages. |
|  |  | Yes(Specify message number) | -change_message=error=[num] | The information-level and warning-level messages with the specified error numbers are changed to error-level messages. |
|  |  | No | --- | Do not change the message level. |
|  | Remark | This property can be used when the check box for [Use same message related settings as Linker page] is not checked. |  |  |

| Number of information-level and warning-level message | Specify the error number for the information-level message or warning-level message to be changed to an error-level message. This corresponds to the -change_message option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify the error number. - Multiple error numbers can be specified by separating them with a comma (,). - A range of error numbers can be specified using a hyphen (-). - Specify the 4-digit number that is output after the component number (05) and the phase of occurrence (6) as the error number (for example, specify 2310 for error number E0562310). - When this property is blank, this option is not output. |
|  | Remark | This property can be used when "Yes(Specify message number)" is selected for [Change information-level and warning-level message to error-level message]. |

4. Converter

# [Description of each category]

## (1) [Output]

This is used to display detailed information on the output control options and make changes to the settings.

| Type of output file | This property is set automatically based on the linker output format defined in [Linker] -> [Output] -> [Type of output file]. |  |
| --- | --- | --- |
|  | Default | Stype via absolute |
|  | How to change | Fixed |
|  | Remark | Read-only |

## (2) [Miscellaneous]

This is used to display detailed information on other items regarding the load module converter and make changes to the settings.

| Use same message related settings as Linker page | This option specifies use of the same message-related settings as specified in the Linker page. This corresponds to the -message, -nomessage, and -change_message options of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | --- | The same message settings as specified in the Linker page are used. |
|  |  | Unchecked | --- | The message options are specified in this page. |
|  | Remark | This property can be used when the box for [Output] -> [Output hex file] is checked. |  |  |

| Enable information-level message output | This option outputs the information-level messages. This corresponds to the -message or -nomessage option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -message | Output information-level messages. |
|  |  | Unchecked | -nomessage | Suppress the output of information-level messages. |
|  | Remark | This property can be used when the check box for [Use same message related settings as Linker page] is not checked. |  |  |

| Suppress number of information-level message | When an error number is specified, the output of the specified information-level message will be disabled. If a warning-level or error-level message number is specified, the message output is disabled assuming that change_message has changed the specified message to an information-level message. This corresponds to the -nomessage option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify the error number. - Multiple error numbers can be specified by separating them with a comma (,). - A range of error numbers can be specified using a hyphen (-). - When this property is blank, the output of all information-level messages is suppressed. - Specify the 4-digit number that is output after the component number (05) and the phase of occurrence (6) as the error number (for example, specify 0004 for error number M0560004). - 0s at the beginning of the 4-digit number can be omitted (for example, specify 4 for error number M0560004). |
|  | Remark | This property can be used when the check box for [Enable information-level message output] is not checked. |

| Change warning-level message to information-level message | This option changes warning-level messages to information-level messages. This corresponds to the -change_message option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes(All) | -change_message=information | All warning-level messages are changed to information-level messages. |
|  |  | Yes(Specify message number) | -change_message=information=[num] | The warning-level messages with the specified error numbers are changed to information-level messages. |
|  |  | No | --- | Do not change the message level. |
|  | Remark | This property can be used when the check box for [Use same message related settings as Linker page] is not checked. |  |  |

| Number of warning-level message | Specify the message number for the warning-level message to be changed to an information-level message. This corresponds to the -change_message option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify the error number. - Multiple error numbers can be specified by separating them with a comma (,). - A range of error numbers can be specified using a hyphen (-). - Specify the 4-digit number that is output after the component number (05) and the phase of occurrence (6) as the error number (for example, specify 2310 for error number E0562310). - When this property is blank, this option is not output. |
|  | Remark | This property can be used when "Yes(Specify message number)" is selected for [Change warning-level message to information-level message]. |

| Change error-level message to warning-level message | This option changes error-level messages to warning-level messages. This corresponds to the -change_message option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes(All) | -change_message=warning | All error-level messages are changed to warning-level messages. |
|  |  | Yes(Specify message number) | -change_message=warning=[num] | The error-level messages with the specified error numbers are changed to warning-level messages. |
|  |  | No | --- | Do not change the message level. |
|  | Remark | This property can be used when the check box for [Use same message related settings as Linker page] is not checked. |  |  |

| Number or error-level message | Specify the error number for the error-level message to be changed to a warning-level message. This corresponds to the -change_message option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify the error number. - Multiple error numbers can be specified by separating them with a comma (,). - A range of error numbers can be specified using a hyphen (-). - Specify the 4-digit number that is output after the component number (05) and the phase of occurrence (6) as the error number (for example, specify 2310 for error number E0562310). - When this property is blank, this option is not output. |
|  | Remark | This property can be used when "Yes(Specify message number)" is selected for [Change error-level message to warning-level message]. |

| Change information-level and warning-level message to error-level message | This option changes information-level and warning-level messages to error-level messages. This corresponds to the -change_message option of the rlink command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes(All) | -change_message=error | All information-level and warning-level messages are changed to error-level messages. |
|  |  | Yes(Specify message number) | -change_message=error=[num] | The information-level and warning-level messages with the specified error numbers are changed to error-level messages. |
|  |  | No | --- | Do not change the message level. |
|  | Remark | This property can be used when the check box for [Use same message related settings as Linker page] is not checked. |  |  |

| Number of information-level and warning-level message | Specify the error number for the information-level message or warning-level message to be changed to an error-level message. This corresponds to the -change_message option of the rlink command. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Specify the error number. - Multiple error numbers can be specified by separating them with a comma (,). - A range of error numbers can be specified using a hyphen (-). - Specify the 4-digit number that is output after the component number (05) and the phase of occurrence (6) as the error number (for example, specify 2310 for error number E0562310). - When this property is blank, this option is not output. |
|  | Remark | This property can be used when "Yes(Specify message number)" is selected for [Change information-level and warning-level message to error-level message]. |

## (3) [Users]

This is used to display detailed information on other items regarding the load module converter and make changes to the settings.

| User-defined options | Specify load module converter options to be added. |  |
| --- | --- | --- |
|  | Default | Blank |
|  | How to change | Use the “AddEbutton to specify the optional character string of rlink command. Use the “DeleteE “EditE “Move UpEor “Move DownEbutton to delete the selected option or to change its name or position in the list. |
|  | Remark | The format of the specified option is not checked. The rlink.exe command load module is executed in the converter phase. The option specified by this property is added as the last of the load module converter options group. |

5. Obj copy

# [Description of each category]

## (1) [Mode]

This is used to display detailed information on the mode options and make changes to the settings.

| Mode | This option controls when the builder will use the standard library to create a new Standard C/C++ library (there is no corresponding command line option). |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Build the library (only when options have been changed) |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Build the library (every time) | --- | A new standard library will be built during every build. Any dependent files will also be rebuilt |
|  |  | Build the library (only when options have been changed) | --- | A new standard library will be built if options have changed or if the user explicitly cleaned the project |
|  |  | Use an existing library file | --- | The specified file should be linked instead of the automatically-generated library file |
|  |  | Do not add a library file | --- | A standard library will be neither generated nor linked; the user has responsibility for manually adding any required libraries in [Linker] -> [Input] -> [Relocatable files, object files and library files] |
|  | Remark | None |  |  |

| User-specified library file | When specified, the designated file will be linked instead of the automatically-generated library file |  |
| --- | --- | --- |
|  | Default | The active build-mode directory of current project. It is in disabled state |
|  | How to change | Use the text box directly to enter the text or use window browser dialog which appears when "Browse" button was clicked. |
|  | Remark | This property can be used when "Use an existing library file" is selected for [Mode]. |

## (2) [Contents]

This is used to display detailed information on the contents options and make changes to the settings.

This category is not displayed when "Use an existing library file" or "Do not add a library file" is specified [Mode] -> [Mode].

| Library configuration | This option selects which functions are to be usable in the C standard library. There are no changes in the functions included in the C++ and EC++ standard libraries. This corresponds to the -lang option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | C (C89) |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | C (C89) | -lang=c | Only the functions conforming to the C89 standard are included in the C standard library, and the extended functions of the C99 standard are not included. |
|  |  | C99 | -lang=c99 | The functions conforming to the C89 standard and the functions conforming to the C99 standard are included in the C standard library. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Runtime routines | Enable runtime library. This corresponds to the -head=runtime option of the lbgrx command. |  |
| --- | --- | --- |
|  | Default | Checked |
|  | How to change | Fixed (Read only) |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |

| Character classification routines | Supply character classifications such as isalnum(), isalpha() and isspace() and corresponding runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=ctype option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=ctype | Enable the ctype.h (C89/C99) and runtime library |
|  |  | Unchecked | --- | Disable the ctype.h (C89/C99) |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Mathematical/ trigonometric operations (double-precision) | Supply mathematical operations mainly for the processing of floating point values with double precision, such as cos(), tan(), and sqrt(), and corresponding runtime routines. When checked, this feature will be built into your standard library This corresponds to the -head=math option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=math | Enable the math.h (C89/C99) and runtime library |
|  |  | Unchecked | --- | Disable the math.h (C89/C99) |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Mathematical/ trigonometric operations (single-precision) | Supply mathematical operations mainly for the processing of floating point values with single precision, such as cosf(), tanf(), and sqrtf(), and corresponding runtime routines. When checked, this feature will be built into your standard library This corresponds to the -head=mathf option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=mathf | Enable the mathf.h (C89/C99) and runtime library |
|  |  | Unchecked | --- | Disable the mathf.h (C89/C99) |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Variable argument functions | Supply support for functions with variable arguments, such as the va_list type and va_arg macro, together with corresponding runtime routines. When checked, this feature will be built into your standard library This corresponds to the -head=stdarg option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=stdarg | Enable the stdarg.h (C89/C99) and runtime library |
|  |  | Unchecked | --- | Disable the stdarg.h (C89/C99) |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Input/ Output | Supply support for IO operations, such as printf(), fopen() and putchar(), together with corresponding runtime routines and the definition of NULL. When checked, this feature will be built into your standard library This corresponds to the -head=stdio option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Checked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=stdio | Enable the stdio.h (C89/C99) and runtime library |
|  |  | Unchecked | --- | Disable the stdio.h (C89/C99) |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| General purpose library features | Supply support for memory allocation, process control, conversations and others, such as malloc(), abort() and atoi(), together with corresponding runtime routines. When checked, this feature will be built into your standard library This corresponds to the -head=stdlib option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Checked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=stdlib | Enable the stdlib.h (C89/C99) and runtime library |
|  |  | Unchecked | --- | Disable the stdlib.h (C89/C99) |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| String handling operations | Supply support for string handling operations, such as strcmp(), strlen() and strcpy() together with corresponding runtime routines. When checked, this feature will be built into your standard library This corresponds to the -head=string option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Checked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=string | Enable the string.h (C89/C99) and runtime library |
|  |  | Unchecked | --- | Disable the string.h (C89/C99) |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Input/ Output streams | Supply the Embedded C++ (EC++) subset of IO operations for C++ applications, such as the ios, streambuf, istream, ostream, iostream and iomanip classes, together with corresponding runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=ios option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=ios | Enable the ios.h (EC++) and runtime library. |
|  |  | Unchecked | --- | Disable the ios.h (EC++). |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Memory allocation and de-allocation routines | Supply the embedded C++ (EC++) subset of memory allocation routines for C++ applications, such as new and delete and supporting runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=new option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Checked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=new | Enable the new.h (EC++) and runtime library. |
|  |  | Unchecked | --- | Disable the new.h (EC++). |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Complex number operations (EC++) | Supply the embedded C++ (EC++) subset of complex number operations for C++ applications, such as float_complex and double_complex classes, together with corresponding operator overloads and runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=complex option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=complex | Enable the complex.h (EC++) and runtime library. |
|  |  | Unchecked | --- | Disable the complex.h (EC++). |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| String manipulation operations | Supply the embedded C++ (EC++) subset of string manipulation operations for C++ applications, such as the String class, together with corresponding operator overloads and runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=cppstring option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=cppstring | Enable the string.h (EC++) and runtime library. |
|  |  | Unchecked | --- | Disable the string.h (EC++). |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Complex number operations (C99) | Supply the support for C99 complex number operations together with corresponding runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=C99_complex option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=C99_complex | Enable the complex.h (C99) and runtime library. |
|  |  | Unchecked | --- | Disable the complex.h (C99). |
|  | Remark | This property is used when "C99" is selected for [Library configuration]. |  |  |

| Floating point operations | Supply the support for C99 floating point operations together with corresponding runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=fenv option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=fenv | Enable the fenv.h (C99) and runtime library. |
|  |  | Unchecked | --- | Disable the fenv.h (C99). |
|  | Remark | This property is used when "C99" is selected for [Library configuration]. |  |  |

| Integer type format operations | Supply the support for C99 integer type format operations together with corresponding runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=inttypes option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=inttypes | Enable the inttypes.h (C99) and runtime library. |
|  |  | Unchecked | --- | Disable the inttypes.h (C99). |
|  | Remark | This property is used when "C99" is selected for [Library configuration]. |  |  |

| Wide character operation | Supply the support for C99 wide character operations together with corresponding runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=wchar option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=wchar | Enable the wchar.h (C99) and runtime library. |
|  |  | Unchecked | --- | Disable the wchar.h (C99). |
|  | Remark | This property is used when "C99" is selected for [Library configuration]. |  |  |

| Wide character classification routines | Supply the support for C99 wide character classification routines together with corresponding runtime routines. When checked, this feature will be built into your standard library. This corresponds to the -head=wctype option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -head=wctype | Enable the wctype.h (C99) and runtime library. |
|  |  | Unchecked | --- | Disable the wctype.h (C99). |
|  | Remark | This property is used when "C99" is selected for [Library configuration]. |  |  |

## (3) [Object]

This is used to display detailed information on the output-related options and make changes to the settings.

| Generate re-entrance library | This option creates reentrant libraries. Note that the rand, srand and EC++ library functions are not reentrant libraries. When reentrant libraries are linked, use #define to define the macro name of _REENTRANT before including standard include files in the program or use the -define option to define _REENTRANT at compilation. This corresponds to the -reent option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -reent | Create the reentrant library. |
|  |  | Unchecked | --- | --- |
|  | Remark | This property can be used when CC-RX V.2.03.00 or a later version is used. |  |  |

| Check memory smashing on releasing memory | This option generates the calloc, free, malloc, and realloc functions with additional security facilities to detect illegal operation on the memory area. When any of the following operations is attempted, the __heap_chk_fail function is called. - A pointer to an area that is not allocated through calloc, malloc, or realloc is passed to free or realloc. - A pointer released through free is again passed to free or realloc. - After a value is written outside the area (within four bytes before and after the area) allocated through calloc, malloc, or realloc, it is passed to free or realloc. The same facility is added to the new and delete operators for C++ programs. The __heap_chk_fail function needs to be defined by the user, and the processing to be executed when an illegal operation for dynamic memory management is found should be written. Note the following when defining the __heap_chk_fail function. - The return value type should always be void, and this function has no parameters. - When defining the __heap_chk_fail function in a C++ program, add <extern "C">. - Take special care so that memory smashing is not detected recursively in the __heap_chk_fail function. This corresponds to the -secure_malloc option of the lbgrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box. |  |  |
|  | Explanation in detail | Checked | -secure_malloc | The alloc, free, malloc, and realloc functions with additional security facilities are generated. |
|  |  | Unchecked | --- | The alloc, free, malloc, and realloc functions without security facilities are generated. |
|  | Remark | This property can be used when CC-RX V2.05.00 or a later version is used. |  |  |

| Output file path | Output the path of standard library file |  |
| --- | --- | --- |
|  | Default | ${CONFIGDIR:/${ProjName}}.lib |
|  | How to change | Fixed (Read only) |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |

| Suppress DIVS, DIVU, and FDIV instruction generation | This option generates code in which no DIV, DIVU, or FDIV instructions are used for division and modular division operations in the program. This option calls the equivalent runtime functions instead of DIV, DIVU, or FDIV instructions. This may lower code efficiency in terms of required ROM capacity and speed of execution. This corresponds to the -nouse_div_inst option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Unchecked | --- | Generate code in which DIV, DIVU, or FDIV instructions are used |
|  |  | Checked | -nouse_div_inst | Generate code in which NO DIV, DIVU, or FDIV instructions are used |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. This property can be specified when CC-RX V1.02.00 or a later version is used. |  |  |

| (Change the section name) Section name of program area | This option specifies the section name of a program area. This corresponds to the -section option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | P |
|  | How to change | Specify the section name (<section name>) of a program area. - <section name> must be alphabetic, numeric, underscore (_), or $. - The first character must not be numeric. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |

| (Change the section name) Section name of constant area | This option specifies the section name of a constant area. This corresponds to the -section option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | C |
|  | How to change | Specify the section name (<section name>) of a constant area. - <section name> must be alphabetic, numeric, underscore (_), or $. - The first character must not be numeric. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |

| (Change the section name) Section name of initialized data area | This option specifies the section name of an initialized data area. This corresponds to the -section option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | D |
|  | How to change | Specify the section name (<section name>) of an initialized data area. - <section name> must be alphabetic, numeric, underscore (_), or $. - The first character must not be numeric. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |

| (Change the section name) Section name of uninitialized data area | This option specifies the section name of an uninitialized data area. This corresponds to the -section option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | B |
|  | How to change | Specify the section name (<section name>) of an uninitialized data area. - <section name> must be alphabetic, numeric, underscore (_), or $. - The first character must not be numeric. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |

| (Change the section name) Section name of switch statement branch table area | This option specifies the section name of a switch statement branch table area. This corresponds to the -section option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | W |
|  | How to change | Specify the section name (<section name>) of a switch statement branch table area. - <section name> must be alphabetic, numeric, underscore (_), or $. - The first character must not be numeric. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |

| (Change the section name) Section name of literal area | This option specifies the section name of a literal area. This corresponds to the -section option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | L |
|  | How to change | Specify the section name (<section name>) of a literal area. - <section name> must be alphabetic, numeric, underscore (_), or $. - The first character must not be numeric. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. This property can be specified when CC-RX V1.02.00 or a later version is used. |

| (Matching the alignment value) Allocates const qualified variable to 4 byte boundary alignment sections | This option allocates const qualified variables to 4-byte boundary alignment sections. This corresponds to the -nostuff option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Unchecked | --- | Allocate to 4-byte, 2-byte, or 1-byte boundary alignment sections depending on the alignment value. |
|  |  | Checked | -nostuff=C | Allocate const qualified variable to 4 byte boundary alignment sections. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| (Matching the alignment value) Allocates initialized variable to 4 byte boundary alignment sections | This option allocates initialized variables to 4-byte boundary alignment sections. This corresponds to the -nostuff option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Unchecked | --- | Allocate to 4-byte, 2-byte, or 1-byte boundary alignment sections depending on the alignment value. |
|  |  | Checked | -nostuff=D | Allocate initialized variable to 4 byte boundary alignment sections |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| (Matching the alignment value) Allocates uninitialized variable to 4 byte boundary alignment sections | This option allocates uninitialized variables to 4-byte boundary alignment sections. This corresponds to the -nostuff option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Unchecked | --- | Allocate to 4-byte, 2-byte, or 1-byte boundary alignment sections depending on the alignment value. |
|  |  | Checked | -nostuff=B | Allocate uninitialized variable to 4 byte boundary alignment sections |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| (Matching the alignment value) Allocates switch statement branch table to 4 byte boundary alignment sections | This option allocates switch statement branch tables to 4-byte boundary alignment sections. This corresponds to the -nostuff option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Unchecked | --- | Allocate to 4-byte, 2-byte, or 1-byte boundary alignment sections depending on the alignment value. |
|  |  | Checked | -nostuff=W | Allocate switch statement branch table to 4 byte boundary alignment sections |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Select adjustment for instruction in branch | This option aligns instructions at branch destinations. This corresponds to the -noinstalign, -instalign4, -instalign8 option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | None |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | None | -noinstalign | Do not align instructions at branch destinations. |
|  |  | Execution in 4 bytes | -instalign4 | Align instructions at branch destinations to 4-byte boundaries. |
|  |  | Execution in 4 bytes(Contains each loop head) | -instalign4=loop | Align instructions at branch destinations to 4-byte boundaries (Contains head of each loop). |
|  |  | Execution in 4 bytes(Contains each inmost loop head) | -instalign4=inmostloop | Align instructions at branch destinations to 4-byte boundaries (Contains head of each inmost loop). |
|  |  | Execution in 8 bytes | -instalign8 | Align instructions at branch destinations to 8-byte boundaries |
|  |  | Execution in 8 bytes(Contains each loop head) | -instalign8=loop | Align instructions at branch destinations to 8-byte boundaries (Contains head of each inmost loop). |
|  |  | Execution in 8 bytes(Contains each inmost loop head) | -instalign8=inmostloop | Align instructions at branch destinations to 8-byte boundaries (Contains head of each inmost loop). |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. This property can be specified when CC-RX V1.01.00 or a later version is used. |  |  |

| Creates a function cutdown version of I/O functions | This option creates a functional cutdown version of I/O functions. This corresponds to the -nofloat, -simple_stdio option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | No |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Yes(Function cutdown version 1) | -nofloat | This option creates simple I/O functions that do not support the conversion of floating-point numbers (%f, %e, %E, %g, %G). |
|  |  | Yes(Function cutdown version 2) | -simple_stdio | The functional cutdown version does not include the conversion of floating-point numbers (same as the function not supported with the nofloat option), the conversion of long long type, and the conversion of 2-byte code. |
|  |  | No | --- | --- |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

## (4) [Optimize]

This is used to display detailed information on the optimization options and make changes to the settings.

| Generate file for inter-module optimization | This option generates the additional information for inter-module optimization in the output file. At linkage, inter-module optimization is applied to files for which this option has been specified. This corresponds to the -goptimize option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -goptimize | Generate file for inter-module optimization |
|  |  | Unchecked | --- | Do not generate file for inter-module optimization |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Optimize level | This option specifies the optimization level. This corresponds to the -optimize option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | 2 |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | 0 | -optimize=0 | The compiler does not optimize the program. Accordingly, the debugging information may be output with high precision and source-level debugging is made easier. |
|  |  | 1 | -optimize=1 | The compiler partially optimizes the program by automatically allocating variables to registers, integrating the function exit blocks, integrating multiple instructions which can be integrated, etc. Accordingly, the code size may become smaller than when compiled with the "0" specification. |
|  |  | 2 | -optimize=2 | The compiler performs overall optimization. However, the optimization contents to be performed slightly differ depending on whether [Optimize type] has been selected. |
|  |  | Max | -optimize=max | The compiler performs optimization as much as possible. For example, the optimization scope is expanded to its maximum extent, and if "Optimize for speed" is selected for [Optimize type], loop expansion is possible on a large scale. Though the advantages of optimization can be expected, there may be side effects, such as longer compilation time, and if "Optimize for speed" is selected for [Optimize type], significantly increased code size. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Optimize type | This option controls the optimization priority, either optimization for speed or optimization for size. This corresponds to the -speed, -size option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Optimize for size |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Optimize for speed | -speed | Optimization will be performed with emphasis on execution performance. |
|  |  | Optimize for size | -size | Optimization will be performed with emphasis on code size. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

## (5) [Optimize]/[Advanced]

This is used to display detailed information on the advanced optimization options and make changes to the settings.

| Switch statement | This option specifies the expansion method of the switch statement. This corresponds to the -case option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Auto |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Auto | -case=auto | The compiler automatically selects the if_then method or table method. |
|  |  | If then | -case=ifthen | The switch statement is expanded using the if_then method, which repeats, for each case label, comparison between the value of the evaluation expression in the switch statement and the case label value. If they match, execution jumps to the statement of the case label. This method increases the object code size depending on the number of case labels in the switch statement. |
|  |  | Table | -case=table | The switch statement is expanded by using the table method, where the case label jump destinations are stored in a branch table so that a jump to the statement of the case label that matches the expression for evaluation in the switch statement is made through a single access to the branch table. With this method, the size of the branch table increases with the number of case labels in the switch statement, but the performance in execution remains the same. The branch table is output to a section for areas holding switch statements for branch tables. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Division by constant | Calculations for division and remainders of integer constants in the source file are converted into sequences of multiplication or bitwise operation (shift or bitwise AND operations) instructions. Constant multiplication that can be performed through only shift operations and division and residue that can be performed through only bitwise AND operations cannot be controlled. This corresponds to the -const_div option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Default |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Default | --- | When "Optimize for speed" is selected for [Optimize] -> [Optimize type], this corresponds to "Multiplication". When "Optimize for size" is selected for [Optimize] -> [Optimize type], this corresponds to "Division". |
|  |  | Multiplication | -const_div | Calculations for division and remainders of integer constants in the source file are converted into sequences of multiplication or bitwise operation (shift or bitwise AND operations) instructions. |
|  |  | Division | -noconst_div | The corresponding division and remainder instructions are used for calculating division and remainders of integer constants in the source file (except divisions and remainders of unsigned integers by powers of two). |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Library function | All library functions are called. This corresponds to the -library option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Inline expansion |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Inline expansion | -library=intrinsic | The instruction expansion is performed for abs(), fabsf(), and library functions which can use string manipulation instructions. |
|  |  | Function call | -library=function | All library functions are called. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Schedule instructions | Instructions are scheduled taking into consideration pipeline processing. This corresponds to the -schedule, -noschedule option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Default |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Default | --- | When "2" or "Max" is selected for [Optimize] -> [Optimize level], this corresponds to "Enable". When "1" or "0" is selected for [Optimize] -> [Optimize level], this corresponds to "Disable". |
|  |  | Enable | -schedule | Instructions are scheduled taking into consideration pipeline processing. |
|  |  | Disable | -noschedule | Instructions are not scheduled. Basically, processing is performed in the same order the instructions have been written in the C/C++ source file. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Divide the optimization | The optimizing ranges of the large-size function are divided into many sections before compilation. Use this option at performance tuning because it affects the object performance depending on the program. This corresponds to the -scope, -noscope option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Default |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Default | --- | When the other than "Max" is selected for [Optimize] -> [Optimize level], this corresponds to "Enable". When "Max" is selected for [Optimize] -> [Optimize level], this corresponds to "Disable". |
|  |  | Enable | -scope | The optimizing ranges of the large-size function are divided into many sections before compilation. |
|  |  | Disable | -noscope | The optimizing ranges are not divided before compilation. When the optimizing range is expanded, the object performance is generally improved although the compilation time is delayed. However, if registers are not sufficient, the object performance may be lowered. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Use floating point arithmetic instructions | A code using FPU instructions is generated. This corresponds to the -fpu, -nofpu option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Default |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Default | --- | When "RX200" or "RX100" is selected for [Compiler] -> [CPU] -> [CPU type], this corresponds to "Disable". In other cases, this corresponds to "Enable". |
|  |  | Enable | -fpu | A code using FPU instructions is generated. |
|  |  | Disable | -nofpu | A code not using FPU instructions is generated. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Loop expansion | This option specifies whether to optimize loop expansion. This corresponds to the -loop option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Default |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Default | --- | When "Optimize for speed" is selected for [Optimize] -> [Optimize type] and "Max" is selected for [Optimize] -> [Optimize level], this corresponds to "-loop=8". When "Optimize for speed" is selected for [Optimize] -> [Optimize type] and "2" is selected for [Optimize] -> [Optimize level], this corresponds to "-loop=2". When "Optimize for speed" is selected for [Optimize] -> [Optimize type] and "0" or "1" is selected for [Optimize] -> [Optimize level], this corresponds to "-loop=1". When "Optimize for size" is selected for [Optimize] -> [Optimize type], this corresponds to "-loop=1". |
|  |  | Custom | -loop=<numeric value> | The compiler expands loop statements (for, while, and do-while). |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Expansion maximum number | The maximum expansion factor can be specified by <numeric value>. This corresponds to the -loop option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | 2 |
|  | How to change | Specify the integer value from 1 to 32. |
|  | Remark | This property can be used when "Custom" is selected for [Loop expansion]. |

| Perform inline expansion automatically | The compiler automatically performs inline expansion. This corresponds to the -inline, -noinline option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Default |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Default | --- | When "Optimize for speed" is selected for [Optimize] -> [Optimize type] and "Max" is selected for [Optimize] -> [Optimize level], this corresponds to "-inline=250". When "Optimize for speed" is selected for [Optimize] -> [Optimize type] and "2" is selected for [Optimize] -> [Optimize level], this corresponds to "-inline=100". When "Optimize for speed" is selected for [Optimize] -> [Optimize type] and "0" or "1" is selected for [Optimize] -> [Optimize level], this corresponds to "None". When "Optimize for size" is selected for [Optimize] -> [Optimize type] and "Max" is selected for [Optimize] -> [Optimize level], this corresponds to "-inline=0". When "Optimize for size" is selected for [Optimize] -> [Optimize type] and "0" or "1" or "2" is selected for [Optimize] -> [Optimize level], this corresponds to "None". |
|  |  | Custom | -inline=<numeric value> | The compiler automatically performs inline expansion. |
|  |  | None | -noinline | The automatic inline expansion is not performed. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Maximum increasing rate of function size | To specify the allowed increase in the function's size due to the use of inline expansion. For example, when "100" is specified, inline expansion will be performed until the function size has increased by 100% (size is doubled). This corresponds to the -inline option of the ccrx command. |  |
| --- | --- | --- |
|  | Default | 100 |
|  | How to change | Specify the integer value from 0 to 65535. |
|  | Remark | This property can be used when "Custom" is selected for [Perform inline expansion automatically]. |

| Performs the constant propagation of const qualified external variables | The constant propagation is performed even for const qualified global variables. This corresponds to the -const_copy,-noconst_copy option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Depends on the optimization level options |  |  |
|  | How to change | Select from the drop-down list |  |  |
|  | Explanation in detail | Depends on the optimization level options | --- | When "Optimize for speed" is selected for [Optimize] -> [Optimize type] and "Max" is selected for [Optimize] -> [Optimize level], this corresponds to "-inline=250". When "Optimize for speed" is selected for [Optimize] -> [Optimize type] and "2" is selected for [Optimize] -> [Optimize level], this corresponds to "-inline=100". When "Optimize for speed" is selected for [Optimize] -> [Optimize type] and "0" or "1" is selected for [Optimize] -> [Optimize level], this corresponds to "None". When "Optimize for size" is selected for [Optimize] -> [Optimize type] and "Max" is selected for [Optimize] -> [Optimize level], this corresponds to "-inline=0". When "Optimize for size" is selected for [Optimize] -> [Optimize type] and "0" or "1" or "2" is selected for [Optimize] -> [Optimize level], this corresponds to "None". |
|  |  | Yes | -const_copy | The constant propagation is performed even for const qualified global variables. |
|  |  | No | -noconst_copy | The constant propagation is disabled for const qualified global variables. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| (Miscellaneous) Treat global variables as volatile qualified | All external variables are handled as if they were volatile qualified. Accordingly, the access count and access order for external variables are exactly the same as those written in the C/C++ source file. This corresponds to the -volatile, -novolatile option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -volatile | All external variables are handled as if they were volatile qualified. |
|  |  | Unchecked | -novolatile | The external variables which are not volatile qualified are optimized. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

## (6) [Miscellaneous]

This is used to display detailed information on the miscellaneous options and make changes to the settings.

| Approximate a floating-point constant division | This option converts divisions of floating-point constants into multiplications of the corresponding reciprocals as constants. To be specific, when there is an expression of (variable / divisor) with the divisor being a constant, a code with the expression converted into (variable × reciprocal of divisor) will be generated. When this option is specified, the execution performance of floating-point constant division will be improved. The precision and order of operations may, however, be changed, so take care on this point. This corresponds to the -approxdiv option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -approxdiv | Convert divisions of floating-point constants into multiplications of the corresponding reciprocals as constants. |
|  |  | Unchecked | --- | Division of floating-point constants into multiplications of the corresponding reciprocals as constants is not performed. |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Enable register declaration | This option allocates preferentially the variables with register storage class specification to registers. This corresponds to the -enable_register option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -enable_register | Allocate preferentially the variables with register storage class specification to registers. |
|  |  | Unchecked | --- | --- |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. This property cannot be specified when CC-RX V2.00.00 or a later version is used. |  |  |

| Not check the range in conversion between floating- point number and integer | This option omits part of the type conversion processing for the floating type. When this option is selected, the generation code that performs type conversion of the next floating-point number changes. - Type conversion from 32-bit floating type to unsigned integer type - Type conversion from unsigned integer type to 32-bit floating type - Type conversion from integer type to 64-bit floating type via 32-bit floating type This corresponds to the -simple_float_conv option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -simple_float_conv | Omit part of the type conversion processing for the floating type. |
|  |  | Unchecked | --- | --- |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. |  |  |

| Change operation order for floating-point expression aggressively | This option optimizes modification of the operation order in a floating-point expression. This corresponds to the -float_order option of the ccrx command. |  |  |  |
| --- | --- | --- | --- | --- |
|  | Default | Unchecked |  |  |
|  | How to change | Select or deselect the check box |  |  |
|  | Explanation in detail | Checked | -float_order | Optimize modification of the operation order in a floating-point expression. |
|  |  | Unchecked | --- | --- |
|  | Remark | This property can be used when "Build the library (every time)" or "Build the library (only when options have been changed)" is selected for [Mode] -> [Mode]. This property cannot be specified when CC-RX V2.00.00 or a later version is used. |  |  |

## (7) [Users]

This is used to display detailed information on other items regarding the compiler and make changes to the settings.

| User-defined options | Specify lbgrx options to be added. |  |
| --- | --- | --- |
|  | Default | --- |
|  | How to change | Use the "Add" button to specify an optional character string of lbgrx command. Use the "Delete", "Edit", "Move Up" or "Move Down" button to delete the selected option or to change its name or position in the list. |
|  | Remark | The format of the specified option is not checked. The option specified by this property is added as the last of the library generator options group. |